##### USER GUIDE

AMX HARMANPRO BSS GATEWAY MODULE (V1.0.14)

BS S AUDI O DE VIC E S / SOUND W E B LOND ON

COPYRIGHT NOTICE

##### AMX© 2020, all rights reserved. No part of this publication may be reproduced, stored in a retrieval system, or transmitted, in any form or by any means, electronic, mechanical, photocopying, recording, or otherwise, without the prior written permission of AMX. Copyright protection claimed extends to AMX hardware and software and includes all forms and matters copyrightable material and information now allowed by statutory or judicial law or herein after granted, including without limitation, material generated from the software programs which are displayed on the screen such as icons, screen display looks, etc. Reproduction or disassembly of embodied computer programs or algorithms is expressly prohibited.

LIABILITY NOTICE

##### No patent liability is assumed with respect to the use of information contained herein. While every precaution has been taken in the preparation of this publication, AMX assumes no responsibility for error or omissions. No liability is assumed for damages resulting from the use of the information contained herein. Further, this publication and features described herein are subject to change without notice.

AMX WARRANTY AND RETURN POLICY

##### The AMX Warranty and Return Policy and related documents can be viewed/downloaded at www.amx.com.

HARDWARE COMPATIBILITY

##### The module has been tested and is compatible on NI and NX series masters expect for the MCP-106/MCP-108 where it is recommended to implement the AMX Soundweb London v2.2 NetLinx module available for download from https://bssaudio.com/en/support_downloads

Table of Contents

HarmanPro Module	6

Overview	6

Module Setup and Communication Process	7

Register Device	7

Register IP Controlled Device	7

Register RS-232 Controlled Device:	7

Subscribe to Objects	9

Overview	9

Channel Feedback	9

Command Feedback	11

Subscribe To Exact Value With Units	11

Subscribe To Percentage Value	13

Update Devices' Value From User Input	13

Level Feedback	13

Subscribe To Level Scaled To Netlinx Standard (0...255)	13

Subscribe To Level Scaled Percentage	15

Subscribe To Level Scaled User Specified Range	17

Bump by Percentage	19

Audio Conferencer	20

Analog Telephone.	20

Voice Over IP Telephone	22

PASSTHRU-	23

RoomCombine Functionality	26

Overview	26

Combining Rooms	26

Creating Partitions	26

Add Room Combine Processing Object	27

Room Combine Processing Object	27

Input and Outputs	28

Chain/Threshold Signal Routing	28

Room Source	28

Background Music (BGM)	28

Room Mix	28

Properties	28

Group Colors	29

Options	29

Room Combine Workflow	30

Register Room Combine	31

Subscribe To One or More Partitions	31

Partition Feedback on Channel	31

Partition Feedback on Command	32

Subscribe to 1 Or More Active Groups (optional)	32

Active Group Feedback on Channel	32

Active Group Feedback on Command	33

Add Room(s)	33

Subscribe to Room objects, Room_Object_Mnemonics	34

Channel Feedback	34

Command Feedback	34

Subscribe to Exact Value With Units	34

Subscribe to Percentage Value	35

Level Feedback	35

Subscribe to Level scaled to NetLinx standard (0...255)	35

Subscribe to Level Scaled Percentage	36

Subscribe to Level Scaled User Specified Range	36

Source Matrix	37

Overview	37

Diagnostics	38

Overview	38

Reporting	39

Report Subscriptions	39

Sample Feedback	39

Room Combine Reporting	40

Partition Report	40

Sample Feedback	40

Room Report	41

Room Combine Complete Report	42

Sample Feedback	42

Telephone Functionality	43

Overview	43

SpeedDial Implementation Notes	43

Analog vs VoIP Differences	43

Feedback Channels	43

Button Actions	43

SpeedDial	44

Update SpeedDial	44

Recall SpeedDial	44

Appendix	45

Object Types	45

RoomCombine Room Object Mnemonics	45

Harman Pro Command Functions	46

Harman Pro Command Data Structures	47

_AddTelephoneStruct	47

_BumpByPercentStruct	47

_PassthruCommandStruct	47

_RecallPreset	47

_RegisterDeviceStruct	47

_RoomCombineAddRoomStruct	48

_RoomCombineActiveGroupSubscribeChannelStruct	48

_RoomCombineActiveGroupSubscribeCommandStruct.	48

_RoomCombinePartitionSubscribeChannelStruct	48

_RoomCombinePartitionSubscribeCommandStruct	48

_RoomCombineRegisterStruct	48

_RoomCombineSubscribeChannelStruct	48

_RoomCombineSubscribeCommandStruct	48

_RoomCombineSubscribeLevelStruct	48

_SpeedDialStruct	48

_SubscribeChannelStruct	49

_SubscribeCommandStruct	49

_SubscribeLevelStruct	49

_UnSubscribeStruct	49

_UpdateCommandValueStruct	49

Dialer NetLinx Module Implemented Buttons	50

VoIP TP	50

POTS TP	52

# REVISION HISTORY

| Date | Comments |
| --- | --- |
| 01/25/2016 JAY | Updated Object Types table |
| 01/13/2016 JAY | Added support for Room Combine, function calls in NetLinx HProRoomCombineCommands.axi |
| 12/01/2016 JAY | Added Support for Analog and VOIP dialers. Replace SEND_COMMAND examples with Function Calls in NetLinx HProCommands.axi |
| 09/06/2016 JAY | Added BR to REGISTER.RS232.DEVICE- Added UNREGISTER.DEVICE- Added ?USAGE |
| 08/06/2017 JAY | Added G4 panels Added logic to detect panel type and send correct command ^UTF ^TXT for Example Changed Dialer keypad buttons to 31xx range Added popup management to HPro Dialer Component.axs rather than touch panel (G5) |
| 13/07/2017 JAY | corrections to TP4 file corrected DTMF tones not processing corrected configuration file for Telephone Input Card not handling Gain (file version 1.2.2) |
| 22/07/2017 JAY | Decreased verbose debug. Fixed issue with poll time property. |
| 04/09/2017 JAY | Resolved Bump requests not being process on ports other than 1. Updated hpconfig.json, v1.2.3 correcting GAN Polarity issue. |
| 17/11/2017 JAY | Created Parameter Preset Recall Example. Added INFO log message indicating what is being sent. |
| 07/03/2018 JAY | Removed references to DIALER.CALLER.ID, DIALER.ERROR.ACCOUNT, DIALER.ERROR.CALL, Removed fields from TP and NL processing, Channels 1, 2, 3 on ports 6 and 7. |
| 19/06/2018 JAY | Resolved xml being dumped, this only occurred when DEBUG-4, it is now removed. |
| 28/08/2018 JAY | Made Room Combine a map rather than a single instance. |
| V1.0.13 PK | Corrected BGM source MAX limit from 2 to 24. BGM Max Limit : 24 hpconfig.json, v1.2.6. |
| V1.0.14 IRC | Re-enable NI compatibility (ITGCommon_1_0_30 PropertyManager). Set Physical-Device=null at NetLinx stub – remove 33 ports being allocated at device 0. Add note re MCP-108 compatibility. |
| V1.0.15 JAY |  |

# HarmanPro Module

## Overview

##### The HarmanPro Module provides the ability for an AMX Central Controller to communicate with one or more BSS Audio devices supporting the Soundweb London 3rd party Control Interface in the Audio Architect application.

##### While the "HPro_BSS_Gateway_Comm_dr1_0_0.jar" is a gateway module, it in itself does not have a real device associated to it. When the module is loaded it provides no control functionality. Using the Register Device function, supply a virtual device number for control of the requested registered device. All control and feedback for the requested device(s) is via that device's virtual device. The gateway module supports 1 to 10 devices. Depending on the number of controls and type of controls, additional Central Controllers may be needed.

##### NetLinx program work flow:

##### DEFINE_MODULE passing the gateway's virtual device.

##### When the ON Channel Event of the gateway's Data Initialized (channel 252) occurs, register one or more devices using the Register Device function with the Device Node Id from Audio Architect with the virtual device, real device (RS-232 control) or IP Address/Hostname.

##### When each device(s) ON Channel Event for Device Online (channel 251) occurs, subscribe to the device objects of interest. Channel, Command and Level control and feedback are supported.

##### Channels should be used with two-state objects such as Mute or Phantom Power.

##### Commands can be used with any object, values will in value and units (if appropriate). For example gain value of 10dB is returned as 10,dB.

##### Levels should be used with objects subscribed to as a percentage, scaled to 0...255 (or user specified range), if the range is not that of the device it will be scaled from device range to the user supplied range.

##### Create Channel, Data and Level events for each subscribed-to object.

##### The required parameters for subscribing to device objects are obtained from Audio Architect. It is recommended that the user use the 3rd Party Controller tool. Detailed examples are included in this document.

##### All of the communications between NetLinx and the Module is accomplished using functions in the "Harman Pro Commands.axi" include file. Each function takes a NetLinx Structure with the values required. Also each function has a corresponding 'Extended' function that takes the individual parameters rather than a structure.

##### The standard Duet module commands, REFRESH, REINIT, DEBUG-[1-4] are supported. If the command is sent to the gateway module, it is forwarded to all registered devices. If sent to a specific registered device, it will only be processed by that device.

##### Registered device virtual devices support PROPERTY, ?PROPERTY and ?PROPERTIES commands.

##### FIG. 1 Audio Architect - Sample Device Parameters

# Module Setup and Communication Process

## Register Device

Device registration must occur after the HPro_BSS_Gateway_dr1_0_0 Data Initialized (Channel 252 ON) Event.

### Register IP Controlled Device

##### Required information retrieved from Audio Architect:

##### IP Address/Hostname

##### Device nodeID, 0xhex format

### Register RS-232 Controlled Device:

##### Required information retrieved from Audio Architect:

##### Device nodeID, 0xhex format

##### Real Device of Master Controllers Serial port

FIG. 2 Audio Architect - Sample Room IP Address/Hostname and nodeID

| INCLUDE 'Harman Pro Commands' | // | Include file of functions to communicate with GateWay Module |  |
| --- | --- | --- | --- |
| DEFINE_DEVICE |  |  |  |
| dvHProIP vdvGateWayModule vdvHProDevice |  | = 0:4:0 = 41001:1:0 = 41010:1:0 |  |
| DEFINE_CONSTANT |  |  |  |
| #WARN 'Verify BSS Audio Device Node Id and IP Address' |  |  |  |
| DEVICE_NODE_ID | = '0x7E0' |  | // Node ID from Audio Architect |
| DEVICE_IP_ADDR | = '10.35.95.16' |  | // IP Address of device |

DEFINE_VARIABLE

_RegisterDeviceStruct	_BSSRegister	// Device register structure DEFINE_START

| _BSSRegister.sNodeID _BSSRegister.vdvDevice | = DEVICE_NODE_ID = vdvHProDevice | // Node ID from Audio Architect // Virtual device to use for the this device |
| --- | --- | --- |
| _BSSRegister.sIPAddrHostname | = DEVICE_IP_ADDR | // Devices IP address |

/*|	|

|	Define Gateway module	|

|	|

| for example: DEFINE_MODULE 'HPro_BSS_Gateway_dr1_0_0' hppro_comm(vdvGateWayModule)	|

|	|

| No REAL DEVICE is specified when defining HPro_BSS_Gateway_Comm_dr1_0_0, it is a gateway module, |

| the base module does not communicate with any device(s).	|

|	|

| When the specified virtual device (vdvGateWayModule) reports Data Initialized (Channel 252 ON)	|

|	which indicates the HPro ConfigFile loaded successfully then you can register devices you're |

|	wanting to control using:	|

|	|

|	fnRegisterIPDevice,	|

|	fnRegisterIPDeviceEx,	|

|	fnRegisterRS232Device,	|

|	fnRegisterRS232DeviceEx or	|

|	fnRegisterRS232DeviceBaudRateEx	|

|	|*/ DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)

DEFINE_EVENT

/*|	|

| Occurs when GateWay module is Data Initialized (Channel 252 ON).	|

|	|

| Use this event to register Device(s)	|

|

CHANNEL_EVENT[vdvGateWayModule, DATA_INITIALIZED]

{

|*/

ON:

{

fnRegisterIPDevice(vdvGateWayModule,_BSSRegister)

}

}

# Subscribe to Objects

## Overview

##### Once the registered devices Data Initialized event occurs the user Subscribes to device objects of interest. Feedback can be a Channel, Command or Level. It is upon the user to determine which feedback methods meets their needs. All device objects will be registered to the requested port on the Devices virtual device.

##### Required information retrieved from Audio Architect:

##### Object ID

##### Parameter ID

##### FIG. 3 Audio Architect - sample Object and Parameter IDs

## Channel Feedback

##### Channel feedback is most applicable for Boolean device objects such as Mute and Power.

PROGRAM_NAME='Subscribe Channel Sample'

INCLUDE 'Harman Pro Commands'	// Include file of functions to communicate with GateWay Module DEFINE_DEVICE

dvHProIP	= 0:4:0

vdvGateWayModule	= 41001:1:0

vdvHProDevice	= 41010:1:0

dvTP	= 10001:1:0

DEFINE_CONSTANT

DEVICE_NODE_ID	= '0x7E0'	// Node ID/Device ID from Audio Architect

DEVICE_IP_ADDR	= '10.35.95.16' // IP Address of device

LONG PRG_DEFINE_TIMES[1]	= {500}	// process TimeLine every 500ms INTEGER DEFINE_PROGRAM_TIMELINE_ID = 6071	// unique TimeLine Id

DEFINE_VARIABLE

| _RegisterDeviceStruct | _BSSRegister | // | Device register structure |
| --- | --- | --- | --- |
| _SubscribeChannelStruct | _Mute | // | Subscribe channel structure |
| VOLATILE INTEGER | nMute = 0 | // | variable to hold value of devices mute status |

DEFINE_START

_BSSRegister.sNodeID	= DEVICE_NODE_ID	// NodeID from Audio Architect

_BSSRegister.vdvDevice	= vdvHProDevice	// Virtual device to use for this device

_BSSRegister.sIPAddrHostname = DEVICE_IP_ADDR	// Devices IP address

_Mute.sObjectID	= '0.0.1'	// ObjectID from Audio Architect

_Mute.sObjectType	= 'AIC'	// Analog Input Card Mnemonic, see documentation for

_Mute.sParameterID	= '0x7D0'	// listing ParameterID from Audio Architect

_Mute.nFBControlPort	= vdvHProDevice.PORT  // Devices' virtual device port for control

_Mute.nFBControlChannel	= GAIN_MUTE_FB	// Channel to report status, i.e. SNAPI Channel for Gain

// Mute Status, 143

/*|	|

|	Define Gateway module	|

|	|

| for example: DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)	|

|	|

| No REAL DEVICE is specified when defining HPro_BSS_Gateway_Comm_dr1_0_0, it is a gateway module, |

|	the base module does not communicate with any device(s).	|

|	|

| When the specified virtual device (vdvGateWayModule) reports Data Initialized (Channel 252 ON)	|

|	which indicates the HPro ConfigFile loaded successfully then you can register devices you're |

|	wanting to control using:	|

|	|

|	fnRegisterIPDevice,	|

|	fnRegisterIPDeviceEx,	|

|	fnRegisterRS232Device,	|

|	fnRegisterRS232DeviceEx or	|

|	fnRegisterRS232DeviceBaudRateEx	|

|	|*/

DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0'	hppro_comm(vdvGateWayModule) DEFINE_EVENT

/*|	|

| Occurs when GateWay module is Data Initialized (Channel 252 ON).	|

|	|

| Use this event to register Device(s)	|

|

CHANNEL_EVENT[vdvGateWayModule, DATA_INITIALIZED]

{

|*/

ON:

{

fnRegisterIPDevice(vdvGateWayModule,_BSSRegister)

}

}

| /*¦ |  | ¦ |
| --- | --- | --- |
| ¦ | When registered device comes Online (Channel 251 ON), Subscribe to device | ¦ |
| ¦ | objects | ¦ |
| ¦ |  | ¦*/ |

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

ON:

{

}

}

/*|

fnSubscribeChannel(vdvHProDevice, _Mute)

|

| Occurs when devices Mute value changes (ON/OFF)	|

|

CHANNEL_EVENT[vdvHProDevice, GAIN_MUTE_FB]

{

|*/

ON:

{

nMute = TRUE

}

OFF:

{

nMute = FALSE

}

}

/*|	|

| Occurs when an user pushes Mute Touch Panel button. Voices update to	|

|	Devices' listening port/channel.	|

|	|*/

BUTTON_EVENT[dvTP, GAIN_MUTE_FB]

{

PUSH:

{

IF(nMute)

{

OFF[vdvHProDevice, GAIN_MUTE_FB]

}

ELSE

{

}

}

/*|

ON[vdvHProDevice, GAIN_MUTE_FB]

}

|

| Due to differences in the underlying architecture of the X-Series masters, |

|	changing variables in the DEFINE_PROGRAM section of code can negatively |

|	impact program performance.	|

|	|

| TimeLine implementation refer to "Differences in DEFINE_PROGRAM Program	|

|	Execution" section of the NX-Series Controllers WebConsole &	|

|	Programming Guide for additional and alternate coding methodologies.	|

|	|*/

TIMELINE_EVENT[DEFINE_PROGRAM_TIMELINE_ID]

{

dvTP, GAIN_MUTE_FB] = nMute

}

## Command Feedback

##### Command feedback supports either exact value with units or percentage.

### Subscribe To Exact Value With Units

PROGRAM_NAME = 'Subscribe Command Sample'

INCLUDE 'Harman Pro Commands'	// Include file of functions to communicate with GateWay Module DEFINE_DEVICE

dvHProIP	= 0:4:0

vdvGateWayModule	= 41001:1:0

vdvHProDevice	= 41010:1:0

dvTP	= 10001:1:0

DEFINE_CONSTANT

#WARN 'Verify BSS Audio Device Node Id and IP Address'

| DEVICE_NODE_ID | = '0x7E0' | // | Node ID from Audio Architect |
| --- | --- | --- | --- |
| DEVICE_IP_ADDR | = '10.35.95.16' | // | IP Address of device |
| FB_GAIN_CMD EDIT_GAIN_CMD TP_EDIT_CHANNEL | = 'FB_GAIN' = 'EDIT_GAIN' = 1 |  |  |

DEFINE_VARIABLE

| _RegisterDeviceStruct | _BSSRegister | // | Device register structure |
| --- | --- | --- | --- |
| _SubscribeCommandStruct | _Command | // | Subscribe Command structure |
| _UpdateCommandValueStruct | _Update | // | Update device command structure |

DEFINE_START

| _BSSRegister.sNodeID _BSSRegister.vdvDevice | = DEVICE_NODE_ID = vdvHProDevice | // // | Node ID from Audio Architect Virtual device to use for the this device |
| --- | --- | --- | --- |
| _BSSRegister.sIPAddrHostname | = DEVICE_IP_ADDR | // | Devices IP address |
| _Command.sObjectID | = '0.0.1' | // | ObjectID from Audio Architect |
| _Command.sObjectType | = 'AIC' | // // | Analog Input Card Mnemonic, see documentation for listing |

| _Command.sParameterID | = '0x4' | // ParameterID from Audio Architect |
| --- | --- | --- |
| _Command.nFBControlPort | = vdvHProDevice.PORT | // Devices' virtual device port for control |
| _Command.sCommandPrefix | = FB_GAIN_CMD | // Command prefix to report status |

/*|	|

|	Define Gateway module	|

|	|

| for example: DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)	|

|	|

| No REAL DEVICE is specified when defining HPro_BSS_Gateway_Comm_dr1_0_0, it is a gateway module, |

|	the base module does not communicate with any device(s).	|

|	|

| When the specified virtual device (vdvGateWayModule) reports Data Initialized (Channel 252 ON)	|

|	which indicates the HPro ConfigFile loaded successfully then you can register devices you're |

|	wanting to control using:	|

|	fnRegisterIPDevice,	|

|	fnRegisterIPDeviceEx,	|

|	fnRegisterRS232Device,	|

|	fnRegisterRS232DeviceEx or	|

|	fnRegisterRS232DeviceBaudRateEx	|

|	|*/

DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule) DEFINE_EVENT

/*|	|

| Occurs when GateWay module is Data Initialized (Channel 252 ON).	|

|	|

| Use this event to register Device(s)	|

|

CHANNEL_EVENT[vdvGateWayModule, DATA_INITIALIZED]

{

|*/

ON:

{

fnRegisterIPDevice(vdvGateWayModule,_BSSRegister)

}

}

| /*¦ |  | ¦ |
| --- | --- | --- |
| ¦ | When registered device comes Online (Channel 251 ON), Subscribe to device | ¦ |
| ¦ | objects | ¦ |
| ¦ |  | ¦*/ |

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

ON:

{

}

}

/*|

fnSubscribeCommand(vdvHProDevice,_Command)

|

| Occurs when an user pushes edit value Touch Panel button. Requests Touch	|

|	Panel Keypad	|

|

BUTTON_EVENT[dvTP, TP_EDIT_CHANNEL]

{

|*/

PUSH:

{

SEND_COMMAND dvTP,"'^AKP-',NULL_STR,';Enter Update Value;',EDIT_GAIN_CMD,'-;1'"

}

}

| /*¦ |  | ¦ |
| --- | --- | --- |
| ¦ | Occurs when device value has changed, update Touch Panel. Format of the | ¦ |
| ¦ | response is sCommandPrefix-value,symbol [if applicable], for example | ¦ |
| ¦ | "FB_GAIN-11.7,dB" or "FB_GAIN-0" | ¦ |
| ¦ |  | ¦*/ |

DATA_EVENT[vdvHProDevice]

{

COMMAND:

{

STACK_VAR CHAR	cCmd[DUET_MAX_CMD_LEN] STACK_VAR CHAR	cHeader[DUET_MAX_HDR_LEN] STACK_VAR CHAR	sValue[DUET_MAX_CMD_LEN] STACK_VAR CHAR	sSymbol[DUET_MAX_CMD_LEN] cCmd = DATA.TEXT

cHeader	= DuetParseCmdHeader(cCmd)

IF (cHeader == FB_GAIN_CMD)

{

sValue	= DuetParseCmdParam(cCmd) sSymbol	= DuetParseCmdParam(cCmd)

// use the ^UTF Set State Text Command using UTF-8 to support symbols (µs) SEND_COMMAND dvTP,"'^UTF-',ITOA(TP_EDIT_CHANNEL),',0,',sValue,sSymbol"

}

}

}

### Subscribe To Percentage Value

fnSubscribeCommandPercent (vdvHProDevice,_Command)

### Update Devices' Value From User Input

/*|	|

| Input from user via Touch Panel, send update to device	|

|

DATA_EVENT[dvTP]

{

STRING:

{

|*/

STACK_VAR CHAR cCmd[DUET_MAX_CMD_LEN] STACK_VAR CHAR cHeader[DUET_MAX_HDR_LEN]

cCmd	= DATA.TEXT

cHeader	= DuetParseCmdHeader(cCmd)

IF (cHeader == EDIT_GAIN_CMD)

{

_Update.sCommandPrefix = FB_GAIN_CMD

_Update.sValue	= DuetParseCmdHeader(cCmd)

fnUpdateCommand(vdvHProDevice,_Update)

}

}

}

## Level Feedback

##### Level feedback supports device values scaled to Level (0 - 255), Percentage (0 - 100) or User specified range.

### Subscribe To Level Scaled To Netlinx Standard (0...255)

PROGRAM_NAME='Subscribe Level NetLinx Range Sample'

INCLUDE 'Harman Pro Commands'	// Include file of functions to communicate with GateWay Module DEFINE_DEVICE

dvHProIP	= 0:4:0

vdvGateWayModule	= 41001:1:0

vdvHProDevice	= 41010:1:1

dvTP	= 10001:1:0

DEFINE_CONSTANT

#WARN 'Verify BSS Audio Device Node Id and IP Address'

| DEVICE_NODE_ID | = '0x7A8' | // Node ID/Device ID from Audio Architect |
| --- | --- | --- |
| DEVICE_IP_ADDR | = '10.35.95.108' | // IP Address of device |
| STEP_BY | = 32 | // Range width 255 / by number of steps 8, // 255/8 == 32 |
| DEFINE_VARIABLE |  |  |
| _RegisterDeviceStruct _BSSRegister |  | // Device register structure |
| _SubscribeLevelStruct _LevelNLRange |  | // Subscribe Level NetLinx range (0...255) structure |
| SINTEGER NLLevelValue |  | // Variable to hold device level value |
| DEFINE_START |  |  |
| _BSSRegister.sNodeID | = DEVICE_NODE_ID | // NodeID from Audio Architect |
| _BSSRegister.vdvDevice | = vdvHProDevice | // Virtual device to use for this device |
| _BSSRegister.sIPAddrHostname | = DEVICE_IP_ADDR | // Devices IP address |
| _LevelNLRange.sObjectID | = '0.0.1' | // ObjectID from Audio Architect |
| _LevelNLRange.sObjectType | = 'AIC' | // Analog Input Card Mnemonic, see documentation // for listing |

| _LevelNLRange.sParameterID _LevelNLRange.nFBControlPort | = '0x04' = vdvHProDevice.PORT | // ParameterID from Audio Architect // Devices' virtual device port for control |
| --- | --- | --- |
| _LevelNLRange.nFBControlLevel | = GAIN_LVL | // SNAPI defined Level (5) to report status |

CREATE_LEVEL dvTP, GAIN_LVL, NLLevelValue

/*|	|

|	Define Gateway module	|

|	|

| for example: DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)	|

|	|

| No REAL DEVICE is specified when defining HPro_BSS_Gateway_Comm_dr1_0_0, it is a gateway module, |

|	the base module does not communicate with any device(s).	|

|	|

| When the specified virtual device (vdvGateWayModule) reports Data Initialized (Channel 252 ON)	|

|	which indicates the HPro ConfigFile loaded successfully then you can register devices you're |

|	wanting to control using:	|

|	|

|	fnRegisterIPDevice,	|

|	fnRegisterIPDeviceEx,	|

|	fnRegisterRS232Device,	|

|	fnRegisterRS232DeviceEx or	|

|	fnRegisterRS232DeviceBaudRateEx	|

|

DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule) DEFINE_EVENT

/*|	|

|*/

| Occurs when GateWay module is Data Initialized (Channel 252 ON).	|

|	|

| Use this event to register Device(s)	|

|

CHANNEL_EVENT[vdvGateWayModule, DATA_INITIALIZED]

{

|*/

ON:

{

fnRegisterIPDevice(vdvGateWayModule,_BSSRegister)

}

}

| /*¦ |  | ¦ |
| --- | --- | --- |
| ¦ | When registered device comes Online (Channel 251 ON), Subscribe to device | ¦ |
| ¦ | objects | ¦ |
| ¦ |  | ¦*/ |

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

ON:

{

}

}

/*|

fnSubscribeLevel(vdvHProDevice,_LevelNLRange)

|

| Occurs when an user pushes GAIN UP Touch Panel button, channel 140.	|

|	Requests increase GAIN by STEP_BY value.	|

| BUTTON_EVENT[dvTP,GAIN_UP]

{

|*/

PUSH:

{

STACK_VAR SINTEGER nVal

nVal = NLLevelValue+STEP_BY IF (nVal > 255)

{

nVal = 255

}

IF (nVal != NLLevelValue)

{

SEND_LEVEL vdvHProDevice,GAIN_LVL,nVal

}

}

}

/*|	|

| Occurs when an user pushes GAIN DOWN Touch Panel button, channel 141.	|

|	Requests decrease GAIN by STEP_BY value.	|

| BUTTON_EVENT[dvTP,GAIN_DN]

{

|*/

PUSH:

{

STACK_VAR SINTEGER nVal

nVal = NLLevelValue-STEP_BY IF (nVal < 0)

{

nVal = 0

}

IF (nVal != NLLevelValue)

{

}

}

/*|

SEND_LEVEL vdvHProDevice,GAIN_LVL,nVal

}

|

| Occurs when devices Gain value changes.  Update Touch Panel level.	|

| LEVEL_EVENT[vdvHProDevice,GAIN_LVL]

{

|*/

SEND_LEVEL dvTP,GAIN_LVL,LEVEL.VALUE

}

### Subscribe To Level Scaled Percentage

PROGRAM_NAME='Subscribe Level Percentage Sample'

INCLUDE 'Harman Pro Commands'	// Include file of functions to communicate with GateWay Module DEFINE_DEVICE

dvHProIP	= 0:4:0

vdvGateWayModule	= 41001:1:0

vdvHProDevice	= 41010:1:1

dvTP	= 10001:1:0

DEFINE_CONSTANT

#WARN 'Verify BSS Audio Device Node Id and IP Address'

| DEVICE_NODE_ID | = | '0x7A8' | // | Node ID/Device ID from Audio Architect |
| --- | --- | --- | --- | --- |
| DEVICE_IP_ADDR | = | '10.35.95.108' | // | IP Address of device |
| BUMP_BY_UP_PRECENT | = | 13 |  |  |
| BUMP_BY_DOWN_PRECENT | = | -7 |  |  |

DEFINE_VARIABLE

_RegisterDeviceStruct _BSSRegister	// Device register structure

_SubscribeLevelStruct _LevelPercent	// Subscribe Level Percentage structure DEFINE_START

| _BSSRegister.sNodeID _BSSRegister.vdvDevice | = = | DEVICE_NODE_ID vdvHProDevice | // // | NodeID from Audio Architect Virtual device to use for this device |
| --- | --- | --- | --- | --- |
| _BSSRegister.sIPAddrHostname | = | DEVICE_IP_ADDR | // | Devices IP address |
| _LevelPercent.sObjectID | = | '0.0.1' | // | ObjectID from Audio Architect |
| _LevelPercent.sObjectType | = | 'AIC' | // // | Analog Input Card Mnemonic, see documentation for listing |
| _LevelPercent.sParameterID | = | '0x04' | // | ParameterID from Audio Architect |
| _LevelPercent.nFBControlPort | = | vdvHProDevice.PORT | // | Devices' virtual device port for control |
| _LevelPercent.nFBControlLevel | = | GAIN_LVL | // | SNAPI defined Level (5) to report status |

/*|	|

|	Define Gateway module	|

|	|

| for example: DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)	|

|	|

| No REAL DEVICE is specified when defining HPro_BSS_Gateway_Comm_dr1_0_0, it is a gateway module, |

|	the base module does not communicate with any device(s).	|

|	|

| When the specified virtual device (vdvGateWayModule) reports Data Initialized (Channel 252 ON)	|

|	which indicates the HPro ConfigFile loaded successfully then you can register devices you're |

|	wanting to control using:	|

|	|

|	fnRegisterIPDevice,	|

|	fnRegisterIPDeviceEx,	|

|	fnRegisterRS232Device,	|

|	fnRegisterRS232DeviceEx or	|

|	fnRegisterRS232DeviceBaudRateEx	|

|	|*/

DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule) DEFINE_EVENT

/*|	|

| Occurs when GateWay module is Data Initialized (Channel 252 ON).	|

|	|

| Use this event to register Device(s)	|

|

CHANNEL_EVENT[vdvGateWayModule, DATA_INITIALIZED]

{

|*/

ON:

{

fnRegisterIPDevice(vdvGateWayModule,_BSSRegister)

}

}

| /*¦ |  | ¦ |
| --- | --- | --- |
| ¦ | When registered device comes Online (Channel 251 ON), Subscribe to device | ¦ |
| ¦ | objects | ¦ |
| ¦ |  | ¦*/ |

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

ON:

{

}

}

/*|

fnSubscribeLevelPercent(vdvHProDevice,_LevelPercent)

|

| Occurs when devices Gain value changes. Update Touch Panel level and text. |

| LEVEL_EVENT[vdvHProDevice,GAIN_LVL]

{

|*/

SEND_LEVEL dvTP,GAIN_LVL,LEVEL.VALUE	// Touch panel level, level code 5, with range of 0...100

}

/*|

// use the ^UTF Set State Text Command using UTF-8 to support symbols (µs) SEND_COMMAND dvTP,"'^UTF-',ITOA(GAIN_LVL),',0,',FTOA(LEVEL.VALUE),'%'"

|

| Occurs when a user pushes GAIN UP Touch Panel button, channel 140.	|

|	Requests increase GAIN by 13%.	|

| BUTTON_EVENT[dvTP,GAIN_UP]

{

|*/

PUSH:

{

fnBumpByPercentageEx(vdvHProDevice,_LevelPercent.sObjectID,_LevelPercent.sParameterID, BUMP_BY_UP_PRECENT)

}

}

/*|	|

| Occurs when a user pushes GAIN DOWN Touch Panel button, channel 141.	|

|	Requests decrease GAIN by 7%.	|

| BUTTON_EVENT[dvTP,GAIN_DN]

{

|*/

PUSH:

{

fnBumpByPercentageEx(vdvHProDevice,_LevelPercent.sObjectID,_LevelPercent.sParameterID, BUMP_BY_DOWN_PRECENT)

}

}

### Subscribe To Level Scaled User Specified Range

PROGRAM_NAME='Subscribe Level Range Sample'

INCLUDE 'Harman Pro Commands'	// Include file of functions to communicate with GateWay Module DEFINE_DEVICE

dvHProIP	= 0:4:0

vdvGateWayModule	= 41001:1:0

vdvHProDevice	= 41010:1:1

dvTP	= 10001:1:0

DEFINE_CONSTANT

#WARN 'Verify BSS Audio Device Node Id and IP Address'

| DEVICE_NODE_ID | = | '0x7A8' | // Node ID/Device ID from Audio Architect |
| --- | --- | --- | --- |
| DEVICE_IP_ADDR | = | '10.35.95.108' | // IP Address of device |

DEFINE_VARIABLE

_RegisterDeviceStruct _BSSRegister	// Device register structure

_SubscribeLevelStruct _LevelRange	// Subscribe Level User defined range structure DEFINE_START

| _BSSRegister.sNodeID _BSSRegister.vdvDevice | = = | DEVICE_NODE_ID vdvHProDevice | // // | NodeID from Audio Architect Virtual device to use for this device |
| --- | --- | --- | --- | --- |
| _BSSRegister.sIPAddrHostname | = | DEVICE_IP_ADDR | // | Devices IP address |
| _LevelRange.sObjectID | = | '0.0.1' | // | ObjectID from Audio Architect |
| _LevelRange.sObjectType | = | 'AIC' | // // | Analog Input Card Mnemonic, see documentation for listing |
| _LevelRange.sParameterID | = | '0x04' | // | ParameterID from Audio Architect |
| _LevelRange.nFBControlPort | = | vdvHProDevice.PORT | // | Devices' virtual device port for control |
| _LevelRange.nFBControlLevel | = | GAIN_LVL | // | SNAPI defined Level (5) to report status |
| _LevelRange.nRangeMinimum | = | 0 | // | Scale to range minimum |
| _LevelRange.nRangeMaximum | = | 8 | // // | Scale to range maximum (gain has 8 steps, stepping by 6 with range of 0, 48 |

/*|	|

|	Define Gateway module	|

|	|

| for example: DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)	|

|	|

| No REAL DEVICE is specified when defining HPro_BSS_Gateway_Comm_dr1_0_0, it is a gateway module, |

|	the base module does not communicate with any device(s).	|

|	|

| When the specified virtual device (vdvGateWayModule) reports Data Initialized (Channel 252 ON)	|

|	which indicates the HPro ConfigFile loaded successfully then you can register devices you're |

|	wanting to control using:	|

|	|

|	fnRegisterIPDevice,	|

|	fnRegisterIPDeviceEx,	|

|	fnRegisterRS232Device,	|

|	fnRegisterRS232DeviceEx or	|

|	fnRegisterRS232DeviceBaudRateEx	|

|

DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)

|*/

DEFINE_EVENT

/*|	|

| Occurs when GateWay module is Data Initialized (Channel 252 ON).	|

|	|

| Use this event to register Device(s)	|

|

CHANNEL_EVENT[vdvGateWayModule, DATA_INITIALIZED]

{

|*/

ON:

{

fnRegisterIPDevice(vdvGateWayModule,_BSSRegister)

}

}

| /*¦ |  | ¦ |
| --- | --- | --- |
| ¦ | When registered device comes Online (Channel 251 ON), Subscribe to device | ¦ |
| ¦ | objects | ¦ |
| ¦ |  | ¦*/ |

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

ON:

{

}

}

/*|

fnSubscribeLevelRange(vdvHProDevice, _LevelRange)

|

| Occurs when an device value changes. Send updated Touch Panel level value. |

| LEVEL_EVENT[vdvHProDevice,GAIN_LVL]

{

|*/

}

/*|

SEND_LEVEL dvTP,GAIN_LVL,LEVEL.VALUE

|

| Occurs when an user changes Touch Panel level, level 5.  Send updated value |

|	to device.	|

| LEVEL_EVENT[dvTP,GAIN_LVL]

{

|*/

SEND_LEVEL vdvHProDevice,GAIN_LVL,LEVEL.VALUE

}

# Bump by Percentage

##### Bump the object by percentage -100 … 100.

##### Usage is not restricted to object subscribed to as percentage.

DEFINE_DEVICE

vdvHProDeviceVolume = 41010:2:0

dvTPVolume	= 10001:2:0 DEFINE_VARIABLE

_BumpByPercentStruct _BumpUp

_BumpByPercentStruct _BumpDown DEFINE_START

_BumpUp.sObjectID	= '0.0.1' // ObjectID from Audio Architect

_BumpUp.sParameterID	= '0x04' // ParameterID from Audio Architect

_BumpUp.fBumpBy	= 13.4	// Bumping Up by 13.4%

_BumpDown.sObjectID	= '0.0.1' // ObjectID from Audio Architect

_BumpDown.sParameterID = '0x04' // ParameterID from Audio Architect

_BumpDown.fBumpBy	= -7	// Bumping Down by 7% DEFINE_EVENT

BUTTON_EVENT[dvTPVolume,VOL_UP]

{

PUSH:

{

fnBumpByPercentage(vdvHProDevice,_BumpUp)

}

}

BUTTON_EVENT[dvTPVolume,VOL_DN]

{ PUSH:

{

fnBumpByPercentage(vdvHProDevice,_BumpDown)

}

}

# Audio Conferencer

##### Implementation of the dialer(s) uses the NetLinx module, 'HPro DialerComponent' refer to the Dialer Functions for supported SNAPI Command and Feedback.

##### Dial Keypad Privacy

##### Volume adjustment Dial number

##### Incoming call announcement Answer call

##### Reject call (VoIP only) Hold call (VoIP only) Redial

##### Auto answer setting

##### Speed dial, recall, save and edit

##### Each dialer uses a unique port on the devices virtual device Each dialer uses a unique port on the Touch panel

## Analog Telephone

PROGRAM_NAME='Dialer POTS Sample'

INCLUDE 'Harman Pro Commands'	// Include file of functions to communicate with GateWay Module DEFINE_DEVICE

| dvHProIP | = 0:4:0 |
| --- | --- |
| vdvGateWayModule | = 41001:1:0 |
| vdvHProDevice | = 41010:1:0 |
| vdvPOTSDialer | = 41010:8:0 |
| dvTP | = 10001:1:0 |
| dvTPPOTSDialer | = 10001:8:0 |

DEFINE_CONSTANT

#WARN 'Verify BSS Audio Device Node Id and IP Address'

| DEVICE_NODE_ID DEVICE_IP_ADDR BTN_BAR_POTS | = '0x7E0' = '10.35.95.16' = 0 | // Node ID/Device ID from Audio Architect // IP Address of device |
| --- | --- | --- |
| SPEEDDIAL_MAX | = 16 | // Range 0...50, should be set to max being used // to conserve resources |
| DEV TPPOTSDialer[1] | = {dvTPPOTSDialer} |  |

DEFINE_VARIABLE

| _RegisterDeviceStruct _AddTelephoneStruct | _BSSRegister _Dialer | // // | Device register structure Subscribed to Telephone structure |
| --- | --- | --- | --- |
| DEVCHAN dcBTN_BAR_POTS | = {dvTP,BTN_BAR_POTS} | // | DEVCHAN for button bars, used only by VoIP |

DEFINE_START

| _BSSRegister.sNodeID _BSSRegister.vdvDevice | = DEVICE_NODE_ID = vdvHProDevice | // // | Node ID from Audio Architect Virtual device to use for the this device |
| --- | --- | --- | --- |
| _BSSRegister.sIPAddrHostname | = DEVICE_IP_ADDR | // | Devices IP address |
| _Dialer.sObjectID | = '0.0.3' | // // | ObjectID of Telephone Input Card from Audio Architect |
| _Dialer.nFBControlPort | = vdvPOTSDialer.PORT | // | Devices' virtual device port for control |
| _Dialer.nSpeedDialMax | = SPEEDDIAL_MAX | // | Number of speed dial slots to support |

/*|	|

|	Define Gateway module	|

|	|

| for example: DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)	|

|	|

| No REAL DEVICE is specified when defining HPro_BSS_Gateway_Comm_dr1_0_0, it is a gateway module, |

|	the base module does not communicate with any device(s).	|

|	|

| When the specified virtual device (vdvGateWayModule) reports Data Initialized (Channel 252 ON)	|

|	which indicates the HPro ConfigFile loaded successfully then you can register devices you're |

|	wanting to control using:	|

|	|

|	fnRegisterIPDevice,	|

|	fnRegisterIPDeviceEx,	|

|	fnRegisterRS232Device,	|

|	fnRegisterRS232DeviceEx or	|

|	fnRegisterRS232DeviceBaudRateEx	|

|

DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)

|*/

| /*¦ |  | ¦ |
| --- | --- | --- |
| ¦ | Implementation of the basic dialer functionality is supplied by the NetLinx module, 'HPro | ¦ |
| ¦ | DialerComponent'.Feedback. | ¦ |
| ¦ | - Dial Keypad | ¦ |
| ¦ | - Privacy | ¦ |
| ¦ | - Volume adjustment | ¦ |
| ¦ | - Dial number | ¦ |
| ¦ | - Incoming call announcement | ¦ |
| ¦ | - Answer call | ¦ |
| ¦ | - Redial | ¦ |
| ¦ | - Auto answer setting | ¦ |
| ¦ | - Speed dial, recall, save and edit | ¦ |
| ¦ |  | ¦*/ |

DEFINE_MODULE 'HPro DialerComponent' dialerpots(vdvPOTSDialer, TPPOTSDialer, dcBTN_BAR_POTS) DEFINE_EVENT

| /*¦ |  | ¦ |
| --- | --- | --- |
| ¦ | When registered module is Data Initialized (Channel 252 ON), Register | ¦ |
| ¦ | device(s) | ¦ |
| ¦ |  | ¦*/ |

CHANNEL_EVENT[vdvGateWayModule, DEVICE_COMMUNICATING]

{

ON:

{

}

}

/*|

fnRegisterIPDevice(vdvGateWayModule,_BSSRegister)

|

| When registered device comes Online (Channel 251 ON), Add telephone line(s) |

|

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

|*/

ON:

{

fnAddPots(vdvHProDevice,_Dialer)

}

}

## Voice Over IP Telephone

##### Dependent upon the BSS devices features 1 or 2 telephone lines are available. The user is required to indicate which telephone line is being subscribed to.

PROGRAM_NAME='Dialer VoIP Sample'

INCLUDE 'Harman Pro Commands'	// Include file of functions to communicate with GateWay Module DEFINE_DEVICE

dvHProIP	= 0:4:0

vdvGateWayModule	= 41001:1:0

vdvHProDevice	= 41010:1:0

vdvVoIPDialer1	= 41010:6:0

dvTP	= 10001:1:0

dvTPVoIPDialer1	= 10001:6:0 DEFINE_CONSTANT

#WARN 'Verify BSS Audio Device Node Id and IP Address'

| DEVICE_NODE_ID DEVICE_IP_ADDR BTN_BAR_VoIP_LINE_1 VoIP_LINE_1 | = '0x7E0' = '10.35.95.16' = 0 = 1 | // Node ID/Device ID from Audio Architect // IP Address of device |
| --- | --- | --- |
| SPEEDDIAL_MAX | = 16 | // Range 0...50, should be set to max being used // to conserve resources |
| DEV TPVoIPDialer1[1] | = {dvTPVoIPDialer1} |  |

DEFINE_VARIABLE

DEVCHAN dcBTN_BAR_VoIP_LINE_1	= {dvTP,BTN_BAR_VoIP_LINE_1}

| _RegisterDeviceStruct | _BSSRegister | // | Device register structure |
| --- | --- | --- | --- |
| _AddTelephoneStruct | _Dialer | // | Subscribed to Telephone structure |

DEFINE_START

| _BSSRegister.sNodeID _BSSRegister.vdvDevice | = DEVICE_NODE_ID = vdvHProDevice | // // | Node ID from Audio Architect Virtual device to use for the this device |
| --- | --- | --- | --- |
| _BSSRegister.sIPAddrHostname | = DEVICE_IP_ADDR | // | Devices IP address |
| _Dialer.sObjectID | = '0.0.3' | // // | ObjectID of Telephone Input Card from Audio Architect |
| _Dialer.nFBControlPort | = vdvVoIPDialer1.PORT | // | Devices' virtual device port for control |
| _Dialer.nTelephoneLine | = VoIP_LINE_1 | // | VoIP Line # |
| _Dialer.nSpeedDialMax | = SPEEDDIAL_MAX | // | Number of speed dial slots to support |

/*|	|

|	Define Gateway module	|

|	|

| for example: DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)	|

|	|

| No REAL DEVICE is specified when defining HPro_BSS_Gateway_Comm_dr1_0_0, it is a gateway module, |

|	the base module does not communicate with any device(s).	|

|	|

| When the specified virtual device (vdvGateWayModule) reports Data Initialized (Channel 252 ON)	|

|	which indicates the HPro ConfigFile loaded successfully then you can register devices you're |

|	wanting to control using:	|

|	|

|	fnRegisterIPDevice,	|

|	fnRegisterIPDeviceEx,	|

|	fnRegisterRS232Device,	|

|	fnRegisterRS232DeviceEx or	|

|	fnRegisterRS232DeviceBaudRateEx	|

|

DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)

|*/

| ¦ | Implementation of the basic dialer functionality is supplied by the NetLinx module, 'HPro | ¦ |
| --- | --- | --- |
| ¦ | DialerComponent'.Feedback. | ¦ |
| ¦ | - Dial Keypad | ¦ |
| ¦ | - Privacy | ¦ |
| ¦ | - Volume adjustment | ¦ |
| ¦ | - Dial number | ¦ |
| ¦ | - Incoming call announcement | ¦ |
| ¦ | - Answer or Reject call | ¦ |
| ¦ | - Hold call | ¦ |
| ¦ | - Redial | ¦ |
| ¦ | - Auto answer setting | ¦ |
| ¦ | - Speed dial, recall, save and edit | ¦ |
| ¦ |  | ¦*/ |

/*|	|

DEFINE_MODULE 'HPro DialerComponent' dialervoip1(vdvVoIPDialer1, TPVoIPDialer1, dcBTN_BAR_VoIP_LINE_1) DEFINE_EVENT

| /*¦ |  | ¦ |
| --- | --- | --- |
| ¦ | When registered module is Data Initialized (Channel 252 ON), Register | ¦ |
| ¦ | device(s) | ¦ |
| ¦ |  | ¦*/ |

CHANNEL_EVENT[vdvGateWayModule, DEVICE_COMMUNICATING]

{

ON:

{

}

}

/*|

fnRegisterIPDevice(vdvGateWayModule,_BSSRegister)

|

| When registered device comes Online (Channel 251 ON), Add telephone line(s) |

|

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

|*/

ON:

{

fnAddVoip(vdvHProDevice,_Dialer)

}

}

## PASSTHRU-

##### Whatever value is forwarded to the device unmodified. Anything received back will be sent to the DPS prefixed with requested text.

###### NOTE: Only HEX Based values supported. The suggested format is AMX.

FIG. 4 Audio Architect - Third Party Controller (AMX selected)

##### Example of setting Analog Output Card - Channel 1 - Mute On

FIG. 5 Audio Architect - example: setting Analog Output Card - Channel 1 - Mute On

PROGRAM_NAME='Passthru Sample' INCLUDE 'Harman Pro Commands'

/*|	|

| Example assumes touch panel with the following buttons	|

| ¦ | Subscribe To Mute: CH Port: 1, CH Code: 1 | ¦ |
| --- | --- | --- |
| ¦ | UnMute: CH Port: 1, CH Code: 2 | ¦ |
| ¦ | Mute: CH Port: 1, CH Code: 3 | ¦ |
| ¦ | Feedback Text: AD Port: 1, AD Code: 10 | ¦ |
| ¦ |  | ¦*/ |

DEFINE_DEVICE

dvHProIP	= 0:4:0

vdvGateWayModule	= 41001:1:0

vdvHProDevice	= 41010:1:0

dvTP	= 10001:1:0

DEFINE_CONSTANT

#WARN 'Verify BSS Audio Device Node Id and IP Address'

| DEVICE_NODE_ID | = '0x7E0' | // Node ID/Device ID from Audio Architect |
| --- | --- | --- |
| DEVICE_IP_ADDR | = '10.35.95.16' | // IP Address of device |
| CMD_MUTE | = 'FB_MUTE' |  |
| CH_TP_FEEDBACK_TEXT | = '10' |  |
| MUTE_SUBSCRIBE MUTE_OFF MUTE_ON | = 1 = 2 = 3 |  |

DEFINE_VARIABLE

| _RegisterDeviceStruct _PassthruCommandStruct | _BSSRegister _PassThruCmds[3] | // Device register structure // Passthru command structure |
| --- | --- | --- |
| DEVCHAN dcBtns[] { |  |  |

{dvTP,MUTE_SUBSCRIBE},

{dvTP,MUTE_OFF},

{dvTP,MUTE_ON}

}

DEFINE_START

_BSSRegister.sNodeID	= DEVICE_NODE_ID	// Node ID from Audio Architect

_BSSRegister.vdvDevice	= vdvHProDevice	// Virtual device to use for this device

//

_BSSRegister.sIPAddrHostname = DEVICE_IP_ADDR	// Devices IP address

/*|	|

| Passthru strings for Subscribe, UnMute and Mute of BLU103 Analog Input	|

|	Card Channel 1 Mute	|

|

_PassThruCmds[MUTE_SUBSCRIBE].nFBControlPort = vdvHProDevice.PORT

_PassThruCmds[MUTE_SUBSCRIBE].sCommandPrefix = CMD_MUTE

_PassThruCmds[MUTE_SUBSCRIBE].sPassthruData	= "'$02,$89,$19,$8B,$1B,$83,$00,$00,$01,$07,$D0,$00,$00,$00,$00,$CE,$03'"

_PassThruCmds[MUTE_ON].nFBControlPort	= vdvHProDevice.PORT

_PassThruCmds[MUTE_ON].sCommandPrefix	= CMD_MUTE

_PassThruCmds[MUTE_ON].sPassthruData	= "'$02,$88,$19,$8B,$1B,$83,$00,$00,$01,$07,$D0,$00,$00,$00,$01,$CE,$03'"

_PassThruCmds[MUTE_OFF].nFBControlPort	= vdvHProDevice.PORT

_PassThruCmds[MUTE_OFF].sCommandPrefix	= CMD_MUTE

_PassThruCmds[MUTE_OFF].sPassthruData	= "'$02,$88,$19,$8B,$1B,$83,$00,$00,$01,$07,$D0,$00,$00,$00,$00,$CF,$03'"

|*/

/*|	|

|	Define Gateway module	|

|	|

| for example: DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule)	|

|	|

| No REAL DEVICE is specified when defining HPro_BSS_Gateway_Comm_dr1_0_0, it is a gateway module, |

|	the base module does not communicate with any device(s).	|

|	|

| When the specified virtual device (vdvGateWayModule) reports Data Initialized (Channel 252 ON)	|

|	which indicates the HPro ConfigFile loaded successfully then you can register devices you're |

|	wanting to control using:	|

|	|

|	fnRegisterIPDevice,	|

|	fnRegisterIPDeviceEx,	|

|	fnRegisterRS232Device,	|

|	fnRegisterRS232DeviceEx or	|

|	fnRegisterRS232DeviceBaudRateEx	|

|	|

|

DEFINE_MODULE 'HPro_BSS_Gateway_Comm_dr1_0_0' hppro_comm(vdvGateWayModule) DEFINE_EVENT

/*|	|

|*/

| Occurs when GateWay module is Data Initialized (Channel 252 ON).	|

|	|

| Use this event to register Device(s)	|

|

CHANNEL_EVENT[vdvGateWayModule, DATA_INITIALIZED]

{

|*/

ON:

{

}

}

/*|

fnRegisterIPDevice(vdvGateWayModule,_BSSRegister)

|

| On button press send button indexed passthru command	|

| BUTTON_EVENT[dcBtns]

{

|*/

PUSH:

{

}

}

/*|

fnPassthruCommand(vdvHProDevice,_PassThruCmds[GET_LAST(dcBtns)])

|

| Occurs when modules sends update from device, if command header equals	|

|	'FB_MUTE' write the commands' text to the touch panel feedback text	|

|	button	|

| DATA_EVENT[vdvHProDevice]

{

|*/

COMMAND:

{

STACK_VAR CHAR	cCmd[DUET_MAX_CMD_LEN] STACK_VAR CHAR	cHeader[DUET_MAX_HDR_LEN]

cCmd = DATA.TEXT

cHeader = UPPER_STRING( DuetParseCmdHeader(cCmd))

IF (cHeader == CMD_MUTE)

{

SEND_COMMAND dvTP,"'^UTF-',CH_TP_FEEDBACK_TEXT,',0,',DATA.TEXT"

}

}

}

# RoomCombine Functionality

## Overview

##### Today, most hotels and conference facilities rely on flexible room configurations for a wide variety of events. Typically this involves one or more large rooms that may be divided by flexible partitions. These combinations of space are referred to as “room combining” and have their own unique requirements when it comes to the audio system. The key is the management of the audio signal to interconnect sub-room sound systems when those sub-rooms are combined into larger rooms.

##### There must be at least two rooms in order to use the Room Combine function. It is assumed here that rooms have been created.

## Combining Rooms

To begin combining rooms, select the Def ine Venue tab (FIG. 6):

FIG. 6 Define Venue tab - Room Combine

##### Select the rooms you wish to combine and click the Create button in the Room Combine section of the ribbon. This will add a Room Combine group in the Venue Explorer. Enter a name for the Room Combine group. Shown below are three groups. In addition, the right-click menu option has a rename function as well (FIG. 7).

FIG. 7 Venue Explorer

## Creating Partitions

##### To add a partition between rooms, select the two rooms and click the Create Partition button. Shown below is a partition between Salon A and Salon B which together are the Blue Room. Partitions may be selected and re-sized (FIG. 8):

FIG. 8 Salon A & B Partitioned to comprise the Blue Room

## Add Room Combine Processing Object

##### Select Add Devices in the Workflow section of the Offline Ribbon and add the BSS BLU device that will be configured with the Room Combine function.

Open the BLU device and select Processing Objects at the bottom of the Venue Explorer.

##### To add a Room Combine group, first select Venue Explorer at the bottom of the Venue Explorer window, then click and drag one of the groups on the left to the BLU device configuration area on the right as shown below (FIG. 9):

FIG. 9 Venue Explorer window

The Room Combine Output Order/Priority dialog will appear (FIG. 10):

FIG. 10 Room Combine Output Order/Priority dialog

##### Select rooms you wish to have moved up or down This will order the rooms from top to bottom in the Room Combine processing object. Note above the rooms outputs are ordered Room 6 at the top, then Room 5, Room 7 and Room 8 at the bottom. Clicking OK brings up the Room Combine processing object.

## Room Combine Processing Object

##### The Room Combine processing object simplifies system setup for such a 'room combining' application. When rooms are "combined", all controls for each involved room will mirror each other perfectly. These controls are objects like faders, mute buttons and source selectors.

FIG. 11 Room Combine Outputs

##### Note that the Room Combine outputs are ordered as directed in the Room Combine Output Order/Priority window.

## Input and Outputs

### Chain/Threshold Signal Routing

##### The Room Combine can route slave automixer control and audio signals. When two or more rooms are involved all automixers will perform as one single automixer to ensure proper gain levels and channel weighting are achieved.

##### These signals are the Chain and Threshold signals located on Slave automixers. Master Automixers or Standalone automixers cannot be used in conjunction with the Room Combine for proper operation. Slave Automixers do not have a Master gain control. When using the Slave Automixer in conjunction with the Room Combine P.O., the Master gain fader is the "Mix" fader located inside The Room Combine P.O (FIG. 12):

FIG. 12 Room Combine Chain/Threshold Signal Routing

### Room Source

##### The Room Source inputs are for receiving a mono audio output from each room. The Source signal input needs to be connected to the output signal of the Slave Automixer if Automixers are utilized.

### Background Music (BGM)

##### The BGM input is used for receiving audio to be distributed in each room as background music. When two or more rooms are combined, the combined rooms will receive the same BGM source.

### Room Mix

##### This output from the P.O. has the mixed audio signal for each room, composed of the room source audio and the BGM audio.

### Properties

FIG. 13 Properties tab

  - Show Type: Show or hide the object type.

  - Show Name: Show or hide the object name.

  - Address: The object address.

##### Name: The object name.

  - HiQnet Address: The object HiQnet address.

##### Type: The object type.

##### Signal Names: Click here to edit the output signal names. Brings up the Signal Naming window.

  - Number of Rooms: Indicates the number of rooms in the group.

  - Number of Partitions: Indicates the number of partitions in the group.

  - Number of BGM Inputs: Sets the number of background music inputs.

### Group Colors

FIG. 14 Group Colors tab

##### The fill color share between rooms in the indicated group. Brings up the Select Color window. This will be the color of the border around the selected group in the Room Combine panel when in Simulate Venue or Run Venue modes

### Options

FIG. 15 Options tab

##### Use Automixer I/O: When checked, the 'Chain' and 'Threshold' signals will be available for wiring with Automixer Gain Sharing and Automixer Gated automixers.

  - Enable stereo functionality: Creates left and right output mixes for each room.

##### Mix BGM Signal Path: When checked, the background music (BGM) signal is mixed with the room mix input.

  - Priority Partition Mode: When checked:

##### Room Combine will operate in 'partition' mode where opening or closing partitions determines which group a room will be part of.

##### When each room is created it will be assigned a 'default group' number which is the same as the room number.

##### When rooms are combined, the group number for the combined room will be set to the lowest numbered room which is present in the room combination.

##### When a room is combined with an existing group of rooms, the room controls will take on the values of the highest priority room (i.e. lowest room number) in the group which is formed.

##### Example 1) Room 1 and Room 2 are combined to form a new room group (group 1). Room 1 master gain is set at 0dB, Room 2 master gain is set at -10dB. At the instant the rooms are combined, both rooms master gain will be set to 0dB.

##### Example 2) Room 1 and Room 2 are already combined in a room group (group 1). Room 3 is combined into the group. Room 1 and 2 master gains are set at 0dB, Room 3 master gain is set at -20dB. At the instant the rooms are combined, all rooms master gains will be set to 0dB.

##### This mode can be useful when you need to perform some logic which is dependent upon knowing the current group that a room is in, e.g. disable the room controls when a room is combined with a higher priority 'master' room. From the design tree you can drag and drop the group (for the room you are checking) onto a logic comparator. Then assign the compare value to be the group number you wish to test against. The logic comparator can then be wired to other objects such as presets, logic ends and so on which can be used to trigger other processing objects or actions. The example shown below can be read as "when room 2 is combined with room 1, i.e. when room 2 becomes part of group 1, do this particular action", and "when room 2 is not combined with room 1, i.e. when room 2 is part of group 2, do this (other) action"

  - When Priority Partition Mode is unchecked -

##### Room Combine may operate in 'room' mode or in 'Partition' mode. In 'room' mode rooms are selected to join a particular group.

##### Each room is created with a default group of 0 ('unassigned').

##### When rooms are combined, the next available group (in sequence 1..number of groups) will be used to determine the group for the room combination.

##### When a room is combined with an existing group of rooms, the room controls for the room group formed will take on the most conservative values of all the rooms in the group which is formed.

##### Example 1) Room 1 and Room 2 are combined to form a new room group (group 1). Room 1 master gain is set at 0dB, Room 2 master gain is set at -10dB. At the instant the rooms are combined, both rooms master gain will be set to -10dB.

##### Example 2) Room 1 and Room 2 are already combined in a room group (group 1). Room 3 is combined into the group. Room 1 and 2 master gains are set at 0dB, Room 3 master gain is set at -20dB. At the instant the rooms are combined, all rooms master gains will be set to -20dB.

##### Changing any of the above properties will cause the default control panel to close.

  - Use Signal Names: When checked, the BGM selectors will use the signal names.

##### Attached to Venue Controls: When checked (default), the Room Combine processing object can be controlled by actions in Room Combine mode. When unchecked, Room combine controls are unattached. This doesn't change the functionality of the Room combine processing object... it only changes how it can be used in room combine mode.

##### This function is used to turn off Room combine functionality, to create a custom User Interface with custom control panels.

###### NOTE: This is an advanced feature. Use only if custom panels are to be created from Room Combine control.

FIG. 16 Audio Architect - Room Combine

## Room Combine Workflow

##### Register Room Combine

##### Subscribe to 1 or more partitions

##### Subscribe to 1 or more Active Groups (optional)

##### Add Room(s)

##### Subscribe to Room objects, Room_Object_Mnemonics

### Register Room Combine

FIG. 17 Audio Architect - Register Room Combine example

DEFINE_DEVICE

vdvHProDevice	= 41010:1:0

vdvHProRoomCombine1	= 41011:1:0

DEFINE_VARIABLE

_RoomCombineRegisterStruct	_RoomCombineRegiser

DEFINE_START

_RoomCombineRegiser.sObjectID	= '0.1.0	// format n.n.n

_RoomCombineRegiser.vdvRoomCombine = vdvHProRoomCombine1 // virtual device Room Combine Object

DEFINE_EVENT

/*|	|

|  When registered device comes Online (Channel 251 ON), Subscribe to device objects |

|

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

|*/

ON:

{

fnRoomCombineRegister(vdvHProDevice,_RoomCombineRegiser)

}

}

### Subscribe To One or More Partitions

FIG. 18 Audio Architect - Subscribe To Partitions example

#### Partition Feedback on Channel

DEFINE_DEVICE

vdvHProDevice	= 41010:1:0

vdvHProRoomCombine1	= 41011:1:0

DEFINE_VARIABLE

_RoomCombinePartitionSubscribeChannelStruct _PartitionChannel

DEFINE_START

_PartitionChannel.sPartitionAddr	= '0x0'	// partition address in 0xhex format

_PartitionChannel.nFBControlPort	= vdvHProRoomCombine1.PORT // virtual device's port to control item

_PartitionChannel.nFBControlChannel = 101	// virtual device's channel to control item

_PartitionChannel.sPartitionName	= 'Room 1 <> Room 2'	// optional, partition name

DEFINE_EVENT

/*|	|

| When registered device comes Online (Channel 251 ON), Subscribe to device objects |

|	|*/

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

ON:

{

fnRoomCombinePartitionSubscribeChannel(vdvHProRoomCombine1,PartitionChannel)

}

}

#### Partition Feedback on Command

DEFINE_DEVICE

vdvHProDevice	= 41010:1:0

vdvHProRoomCombine1	= 41011:1:0

DEFINE_VARIABLE

_RoomCombinePartitionSubscribeCommandStruct _PartitionCommand

DEFINE_START

_PartitionCommand.sPartitionAddr = '0x1'	// partition address in 0xhex format

_PartitionCommand.nFBControlPort = vdvHProRoomCombine1.PORT // virtual device's port to control item

_PartitionCommand.sCommandPrefix = 'FB_PART'	// command prefix

_PartitionCommand.sPartitionName = 'Room 2 <> Room 3'	// optional, partition name

DEFINE_EVENT

/*|	|

|  When registered device comes Online (Channel 251 ON), Subscribe to device objects	|

|	|*/

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

ON:

{

fnRoomCombinePartitionSubscribeCommand(vdvHProRoomCombine1,PartitionCommand)

}

}

### Subscribe to 1 Or More Active Groups (optional)

FIG. 19 Audio Architect - Subscribe To Active Groups example

#### Active Group Feedback on Channel

DEFINE_DEVICE

vdvHProDevice	= 41010:1:0

vdvHProRoomCombine1	= 41011:1:0

DEFINE_VARIABLE

_RoomCombineActiveGroupSubscribeChannelStruct_ActiveGroupChannel

DEFINE_START

_ActiveGroupChannel.sActiveGroupAddr ='0xF9	// active group address in 0xhex format

_ActiveGroupChannel.nFBControlPort	= vdvHProRoomCombine1.PORT // virtual device's port to control item

_ActiveGroupChannel nFBControlChannel = 100	// virtual device's channel to control item

DEFINE_EVENT

/*|	|

|  When registered device comes Online (Channel 251 ON), Subscribe to device objects	|

|

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

|*/

ON:

{

fnRoomCombineActiveGroupSubscribeChannel(vdvHProRoomCombine1, _ActiveGroupChannel)

}

}

#### Active Group Feedback on Command

DEFINE_DEVICE

vdvHProDevice	= 41010:1:0

vdvHProRoomCombine1	= 41011:1:0

DEFINE_VARIABLE

_RoomCombineActiveGroupSubscribeCommandStruct_ActiveGroupCommand

DEFINE_START

_ActiveGroupCommand.sActiveGroupAddr = '0xF9'	// active group address in 0xhex format

_ActiveGroupCommand.nFBControlPort	= vdvHProRoomCombine1.PORT	// virtual device's port

_ActiveGroupCommand.sCommandPrefix	= 'FB_AGRP'	// command prefix

DEFINE_EVENT

/*|	|

|  When registered device comes Online (Channel 251 ON), Subscribe to device objects	|

|	|*/

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

ON:

{

fnRoomCombineActiveGroupSubscribeCommand(vdvHProRoomCombine1, _ActiveGroupCommand)

}

}

### Add Room(s)

FIG. 20 Audio Architect - Add Room example

DEFINE_DEVICE

vdvHProDevice	= 41010:1:0

vdvHProRoomCombine1	= 41011:1:0

DEFINE_VARIABLE

_RoomCombineAddRoomStruct _RoomAdd

DEFINE_START

_RoomAdd.sRoomGroupAddr	= '0x12C'	// room group address in 0xhex format

_RoomAdd.sRoomName	= 'Room 1' // optional, room name

DEFINE_EVENT

/*|	|

|  When registered device comes Online (Channel 251 ON), Subscribe to device objects |

|	|*/

CHANNEL_EVENT[vdvHProRoomCombine1, DEVICE_COMMUNICATING]

{

ON:

{

fnRoomCombineAddRoom(vdvHProRoomCombine1,_RoomAdd[1])

}

}

### Subscribe to Room objects, Room_Object_Mnemonics

Subscription to Room object must occur after calling the fnRoomCombineAddRoom(Ex) function

FIG. 21 Subscribe to Room objects

## Channel Feedback

##### Channel feedback is most applicable for Boolean device objects such as Mute, Power…

DEFINE_DEVICE

vdvHProDevice	= 41010:1:0

vdvHProRoomCombine1	= 41011:1:0

DEFINE_VARIABLE

_RoomCombineSubscribeChannelStruct _RCSubscribeChannel

DEFINE_START

_RCSubscribeChannel.sRoomGroupAddr	= '0x12C'	// room group address in 0xhex format

_RCSubscribeChannel.sRoomItemMnemonic = 'MM'	// Master Mute. see documentation

_RCSubscribeChannel.nFBControlPort	= vdvHProRoomCombine1.PORT // virtual device's port to control item

_RCSubscribeChannel.nFBControlChannel = VOL_MUTE	// virtual device's channel to control item

DEFINE_EVENT

/*|	|

|  When registered device comes Online (Channel 251 ON), Subscribe to device objects	|

|	|*/

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

ON:

{

fnRoomCombineRoomSubscribeChannel(vdvHProRoomCombine1,_RCSubscribeChannel)

}

}

## Command Feedback

##### Command feedback supports either exact value with units or percentage.

### Subscribe to Exact Value With Units

DEFINE_DEVICE

vdvHProDevice	= 41010:1:0

vdvHProRoomCombine1	= 41011:1:0

DEFINE_VARIABLE

_RoomCombineSubscribeCommandStruct	_RCSubscribeCommand

DEFINE_START

_RCSubscribeCommand.sRoomGroupAddr	= '0x12C'	// room group address in 0xhex format

_RCSubscribeCommand.sRoomItemMnemonic = 'BS'	// BGM Source, see documentation

_RCSubscribeCommand.nFBControlPort	= vdvHProRoomCombine1.PORT // virtual device's port to control item

_RCSubscribeCommand.sCommandPrefix	= 'Room1BGMSource'	// command prefix

DEFINE_EVENT

/*|	|

|  When registered device comes Online (Channel 251 ON), Subscribe to device objects	|

|

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

|*/

{

ON:

{

fnRoomCombineRoomSubscribeCommand(vdvHProRoomCombine1,_RCSubscribeCommand)

}

}

### Subscribe to Percentage Value

DEFINE_DEVICE

vdvHProDevice	= 41010:1:0

vdvHProRoomCombine1	= 41011:1:0

DEFINE_VARIABLE

_RoomCombineSubscribeCommandStruct _RCSubscribeCommand

DEFINE_START

_RCSubscribeCommand.sRoomGroupAddr	='0x12C'	// room group address in 0xhex format

_RCSubscribeCommand.sRoomItemMnemonic ='MG'	// Master Gain, see documentation

_RCSubscribeCommand.nFBControlPort	= vdvHProRoomCombine1.PORT // virtual device's port to control item

_RCSubscribeCommand.sCommandPrefix	= 'Room1MasterGain'	// command prefix

DEFINE_EVENT

/*|	|

|  When registered device comes Online (Channel 251 ON), Subscribe to device objects	|

|

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

ON:

{

fnRoomCombineRoomSubscribeCommandPercent(vdvHProRoomCombine1,_RCSubscribeCommand)

}

|*/

}

## Level Feedback

##### Level feedback supports device values scaled to Level (0 - 255), Percentage (0 - 100) or User specified range.

### Subscribe to Level scaled to NetLinx standard (0...255)

DEFINE_CONSTANT

GAIN_LEVEL	= 33

DEFINE_DEVICE

vdvHProDeviceVolume	= 41010:2:0

vdvHProRoomCombine1	= 41011:1:0

dvTPVolume	= 10001:2:0

DEFINE_VARIABLE

_RoomCombineSubscribeLevelStruct_RCSubscribeLevel

DEFINE_START

_RCSubscribeLevel.sRoomGroupAddr	='0x12C'	// room group address in 0xhex format

_RCSubscribeLevel.sRoomItemMnemonic ='SG'	// Source Gain, see documentation

_RCSubscribeLevel.nFBControlPort	= vdvHProRoomCombine1.PORT // virtual device's port to control item

_RCSubscribeLevel.nFBControlLeve	= GAIN_LEVEL	// Level to report status

DEFINE_EVENT

/*|	|

|  When registered device comes Online (Channel 251 ON), Subscribe to device objects	|

|

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

ON:

{

fnRoomCombineRoomSubscribeLevel(vdvHProRoomCombine1, _RCSubscribeLevel)

}

|*/

### Subscribe to Level Scaled Percentage

DEFINE_CONSTANT

GAIN_LEVEL	= 33

DEFINE_DEVICE

vdvHProDeviceVolume	= 41010:2:0

vdvHProRoomCombine1	= 41011:1:0

dvTPVolume	= 10001:2:0

DEFINE_VARIABLE

_RoomCombineSubscribeLevelStruct_RCSubscribeLevel

DEFINE_START

_RCSubscribeLevel.sRoomGroupAddr	='0x12C'	// room group address in 0xhex format

_RCSubscribeLevel.sRoomItemMnemonic ='SG'	// Source Gain, see documentation

_RCSubscribeLevel.nFBControlPort	= vdvHProRoomCombine1.PORT // virtual device's port to control item

_RCSubscribeLevel.nFBControlLeve	= GAIN_LEVEL	// Level to report status

DEFINE_EVENT

/*|	|

|  When registered device comes Online (Channel 251 ON), Subscribe to device objects	|

|

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

|*/

{

ON:

{

}

}

/*|

fnRoomCombineRoomSubscribeLevelPercent(vdvHProRoomCombine1, _RCSubscribeLevel)

|

| Update touch panel when device value changes	|

|

LEVEL_EVENT[vdvHProRoomCombine1,GAIN_LEVEL]

|*/

{

SEND_LEVEL dvTPVolume,GAIN_LEVEL,LEVEL.VALUE

}

### Subscribe to Level Scaled User Specified Range

DEFINE_CONSTANT GAIN_LEVEL= 33

DEFINE_DEVICE

vdvHProDeviceVolume	= 41010:2:0

vdvHProRoomCombine1	= 41011:1:0

dvTPVolume	= 10001:2:0

DEFINE_VARIABLE

_RoomCombineSubscribeLevelStruct	_RCSubscribeLevel

DEFINE_START

_RCSubscribeLevel.sRoomGroupAddr	='0x12C'	// room group address in 0xhex format

_RCSubscribeLevel.sRoomItemMnemonic ='SG'	// Source Gain, see documentation

_RCSubscribeLevel.nFBControlPort	= vdvHProRoomCombine1.PORT // virtual device's port to control item

_RCSubscribeLevel.nFBControlLeve	= GAIN_LEVEL	// Level to report status

_RCSubscribeLevel.nRangeMinimum	= -80	// level range minimum

_RCSubscribeLevel.nRangeMaximum	= 10	// level range maximum

DEFINE_EVENT

/*|	|

|  When registered device comes Online (Channel 251 ON), Subscribe to device objects	|

|

CHANNEL_EVENT[vdvHProDevice, DEVICE_COMMUNICATING]

{

|*/

ON:

{

fnRoomCombineRoomSubscribeLevelRange(vdvHProRoomCombine1,_RCSubscribeLevel)

}

}

# Source Matrix

## Overview

##### The Source Matrix is one of widely used BSS Audio objects.

##### The "Source Matrix Example" system NetLinx project demonstrates how it might be implemented.

# Diagnostics

## Overview

##### Channel 256 controls Heartbeat and Polling events, turning 256 ON disables these events. Turning it on the Gateway's virtual disables it for all devices turning on a specific virtual affects only that devices.

##### Enable logging within the NetLinx "HPro Commands.axi" using SEND_C 33099,"'DEBUG-[0-4]'" where

DEBUG_OFF	= 0

DEBUG_ERROR	= 1

DEBUG_WARN	= 2

DEBUG_INFO	= 3

DEBUG_DEBUG	= 4

##### To query the status of logging SEND_C 33099,"'?DEBUG'"

##### Enabling DEBUG_DEBUG will result in all data sent to the Network Controller to be written to a log file, "HProSendCommandsLog.xml".

##### You can add logging to your NetLinx program using the fnDebugMessage(msg_level, message). Where msg_level is DEBUG_OFF, DEBUG_ERROR, DEBUG_WARN, DEBUG_INFO or DEBUG_DEBUG.

# Reporting

## Report Subscriptions

##### Reports all items by subscribed to for specified virtual device, if module all registered devices are reported.

PROGRAM_NAME ='Reporting Subscriptions'

INCLUDE 'SNAPI'

INCLUDE 'Harman Pro Commands'

DEFINE_DEVICE

vdvHProModule	= 41001:1:0

dvTPReporting	= 10001:1:0

DEFINE_CONSTANT

BTN_REPORT_SUB	= 1

DEFINE_EVENT

DATA_EVENT[vdvHProModule]

{

COMMAND:

{

STACK_VAR CHAR	cCmd[DUET_MAX_CMD_LEN] STACK_VAR CHAR	cHeader[DUET_MAX_HDR_LEN]

STACK_VAR INTEGER rpt

rpt = FALSE

cCmd = DATA.TEXT

cHeader	= DuetParseCmdHeader(cCmd)

SWITCH (cHeader)

{

CASE CMD_SUBSCRIPTIONS_REPORT:

{

SEND_STRING 0,DATA.TEXT

}

}

}

}

BUTTON_EVENT[dvTPReporting,BTN_REPORT_SUB]

{

PUSH:

{

fnReportSubscriptions(vdvHProModule)

}

}

#### Sample Feedback

REPORT.SUBSCRIPTIONS-41010:1:1,sNodeID=0x1989,vdvDevice=41010,sIPAddrHostname=10.35.95.98

REPORT.SUBSCRIPTIONS-41010:3:1,SubscribeCommand,sCommandPrefix=FB_ATTACK_CH3,AIC,Analog Input Card,parametername=Attack,parameterid=0x000E,virtualdeviceindex=0x03,defaultvalue=10.00$B5s,min=10.00$B5s, max=200.00s

REPORT.SUBSCRIPTIONS-41010:3:1,SubscribeCommand,sCommandPrefix=FB_ATTACK_CH4,AIC,Analog Input Card,parametername=Attack,parameterid=0x0014,virtualdeviceindex=0x03,defaultvalue=10.00$B5s,min=10.00$B5s, max=200.00s

REPORT.SUBSCRIPTIONS-41010:4:1,SubscribeLevelPercent,nFBControlLevel=35,AIC,Analog Input Card,parametername=Release,parameterid=0x000F,virtualdeviceindex=0x03,defaultvalue=50.00s,min=50.00s,max=5.00s REPORT.SUBSCRIPTIONS-41010:4:1,SubscribeLevelPercent,nFBControlLevel=36,AIC,Analog Input Card,parametername=Release,parameterid=0x0015,virtualdeviceindex=0x03,defaultvalue=50.00s,min=50.00s,max=5.00s

REPORT.SUBSCRIPTIONS-41010:4:1,SubscribeLevelRange,nFBControlLevel=38,nRangeMinimum=0.0,nRangeMaximum=8.0, AIC,Analog Input Card,parametername=Gain,parameterid=0x000A,virtualdeviceindex=0x03,defaultvalue=0.00dB, min=0.00dB,max=+48.00dB

REPORT.SUBSCRIPTIONS-41010:4:1,SubscribeLevelRange,nFBControlLevel=39,nRangeMinimum=0.0,nRangeMaximum=8.0, AIC,Analog Input Card,parametername=Gain,parameterid=0x0010,virtualdeviceindex=0x03,defaultvalue=0.00dB, min=0.00dB,max=+48.00dB

REPORT.SUBSCRIPTIONS-41010:4:1,SubscribeLevelRange,nFBControlLevel=40,nRangeMinimum=0.0,nRangeMaximum=8.0, AIC,Analog Input Card,parametername=Gain,parameterid=0x0016,virtualdeviceindex=0x03,defaultvalue=0.00dB, min=0.00dB,max=+48.00dB

## Room Combine Reporting

### Partition Report

PROGRAM_NAME='Reporting Room Combine Partition'

INCLUDE 'SNAPI'

INCLUDE 'Harman Pro Commands'

DEFINE_DEVICE

vdvReportRoomCombine	= 41012:1:0

dvTPReporting	= 10001:1:0

DEFINE_CONSTANT

BTN_REPORT_RC_PART	= 3

DEFINE_EVENT

DATA_EVENT[vdvReportRoomCombine]

{

COMMAND:

{

STACK_VAR CHAR	cCmd[DUET_MAX_CMD_LEN] STACK_VAR CHAR	cHeader[DUET_MAX_HDR_LEN]

cCmd = DATA.TEXT

cHeader	= DuetParseCmdHeader(cCmd)

SWITCH (cHeader)

{

CASE CMD_RC_PARTITION_REPORT:

{

SEND_STRING 0,DATA.TEXT

}

}

}

}

BUTTON_EVENT[dvTPReporting,BTN_REPORT_RC_PART]

{

PUSH:

{

fnReportRoomCombinePartition(vdvReportRoomCombine)

}

}

#### Sample Feedback

REPORT.ROOMCOMBINE.PARTITION-41012:1:1,sPartitionName=Room 1 <> Room 2,sPartitionAddr=0x0 REPORT.ROOMCOMBINE.PARTITION-41012:1:1,sPartitionName=Room 1 <> Room 3,sPartitionAddr=0x2 REPORT.ROOMCOMBINE.PARTITION-41012:1:1,sPartitionName=Room 2 <> Room 3,sPartitionAddr=0x1

### Room Report

PROGRAM_NAME=' Reporting Room Combine Room'

INCLUDE 'SNAPI'

INCLUDE 'Harman Pro Commands'

DEFINE_DEVICE

vdvReportRoomCombine	= 41012:1:0

dvTPReporting	= 10001:1:0

DEFINE_CONSTANT

BTN_REPORT_RC_ROOM	= 4

DEFINE_EVENT

DATA_EVENT[vdvReportRoomCombine]

{

COMMAND:

{

STACK_VAR CHAR	cCmd[DUET_MAX_CMD_LEN] STACK_VAR CHAR	cHeader[DUET_MAX_HDR_LEN]

cCmd = DATA.TEXT

cHeader	= DuetParseCmdHeader(cCmd)

SWITCH (cHeader)

{

CASE CMD_RC_ROOM_REPORT:

{

SEND_STRING 0,DATA.TEXT

}

}

}

}

BUTTON_EVENT[dvTPReporting,BTN_REPORT_RC_ROOM]

{

PUSH:

{

fnReportRoomCombineRoom(vdvReportRoomCombine)

}

}

Sample Feedback

REPORT.ROOMCOMBINE.ROOM-41012:1:1,"sRoomGroupAddr:0x12C,sRoomName=Room 1,GP=0x12C,IA=0x12D, MG=0x12E,MM=0x130,SG=0x131,SM=0x132,BG=0x133,BM=0x134,BS=0x135"

REPORT.ROOMCOMBINE.ROOM-41012:1:1,"sRoomGroupAddr:0x15E,sRoomName=Room 3,GP=0x15E,IA=0x15F, MG=0x160,MM=0x162,SG=0x163,SM=0x164,BG=0x165,BM=0x166,BS=0x167"

REPORT.ROOMCOMBINE.ROOM-41012:1:1,"sRoomGroupAddr:0xFA,sRoomName=Room 2,GP=0xFA,IA=0xFB, MG=0xFC,MM=0xFE,SG=0xFF,SM=0x100,BG=0x101,BM=0x102,BS=0x103

Room Combine Complete Report Reports combines Partition and Room report. PROGRAM_NAME= Reporting Room Combine'

INCLUDE 'SNAPI'

INCLUDE 'Harman Pro Commands'

DEFINE_DEVICE

vdvReportRoomCombine	= 41012:1:0

dvTPReporting	= 10001:1:0

DEFINE_CONSTANT

BTN_REPORT_RC	= 2

DEFINE_EVENT

DATA_EVENT[vdvReportRoomCombine]

{

COMMAND:

{

STACK_VAR CHAR	cCmd[DUET_MAX_CMD_LEN] STACK_VAR CHAR	cHeader[DUET_MAX_HDR_LEN]

cCmd = DATA.TEXT

cHeader	= DuetParseCmdHeader(cCmd) SWITCH (cHeader)

{

CASE CMD_RC_REPORT:

{

SEND_STRING 0,DATA.TEXT

}

}

}

}

BUTTON_EVENT[dvTPReporting,BTN_REPORT_RC]

{

PUSH:

{

fnReportRoomCombine(vdvReportRoomCombine)

}

}

Sample Feedback

REPORT.ROOMCOMBINE.PARTITION-41012:1:1,sPartitionName=Room 1 <> Room 2,sPartitionAddr=0x0 REPORT.ROOMCOMBINE.PARTITION-41012:1:1,sPartitionName=Room 1 <> Room 3,sPartitionAddr=0x2 REPORT.ROOMCOMBINE.PARTITION-41012:1:1,sPartitionName=Room 2 <> Room 3,sPartitionAddr=0x1

REPORT.ROOMCOMBINE.ROOM-41012:1:1,"sRoomGroupAddr:0x12C,sRoomName=Room 1,GP=0x12C,IA=0x12D, MG=0x12E,MM=0x130,SG=0x131,SM=0x132,BG=0x133,BM=0x134,BS=0x135"

REPORT.ROOMCOMBINE.ROOM-41012:1:1,"sRoomGroupAddr:0x15E,sRoomName=Room 3,GP=0x15E,IA=0x15F, MG=0x160,MM=0x162,SG=0x163,SM=0x164,BG=0x165,BM=0x166,BS=0x167"

REPORT.ROOMCOMBINE.ROOM-41012:1:1,"sRoomGroupAddr:0xFA,sRoomName=Room 2,GP=0xFA,IA=0xFB, MG=0xFC,MM=0xFE,SG=0xFF,SM=0x100,BG=0x101,BM=0x102,BS=0x103"

# Telephone Functionality

## Overview

##### A NetLinx module is provided that implements the basic functionality of the Telephone cards, HPro Dialer Complent.axs.

### SpeedDial Implementation Notes

##### Telephone Input Card (Analog/POTS) does not support storing or recalling the SpeedDial name. The VoIP Input Card does.

##### Use of the max SpeedDial entries reduces resources.

## Analog vs VoIP Differences

|  | Analog | VoIP |
| --- | --- | --- |
| Reject Call |  |  |
| + key |  |  |
| Flash |  |  |
| Hold |  |  |
| Caller ID |  |  |

## Feedback Channels

| Channel | Constant |
| --- | --- |
| 85 | CH_DIALER_HOLD |
| 202 | CH_DIALER_OFF_HOOK |
| 201 | CH_DIALER_REDIAL |
| 61 | CH_DIALER_REJECT |
| 257 | CH_DIALER_INCOMING_CALL |
| 258 | CH_DIALER_LINE_STATUS |
| 239 | CH_DIALER_AUTO_ANSWER |
| 240 | CH_DIALER_AUDIBLE_RING |
| 146 | CH_DIALER_PRIVACY |

### Button Actions

| Channel | Constant | Usage |
| --- | --- | --- |
| 24 | VOL_UP | Increments the RX volume by 10%, nBumpByPercentageEx(vdvDev,NULL_STR,NULL_STR,10) |
| 25 | VOL_DN | Decrements the RX volume by 10%, fnBumpByPercentageEx(vdvDev,NULL_STR,NULL_STR,-10) |
| 61 | MENU_REJECT | fnUpdateCommandEx(vdvDev, CMD_DIALER_REJECT_CALL,NULL_STR) |
| 85 | MENU_HOLD | fnUpdateCommandEx(vdvDev, CMD_DIALER_HOLD,NULL_STR) |
| 146 | ACONF_PRIVACY_ON | Toggle, fnUpdateCommandEx(vdvDev,CMD_DIALER_PRIVACY,ITOA(!bPrivacy)) |
| 201 | DIAL_REDIAL | PULSE[vdvDev,DIAL_REDIAL] |
| 239 | DIAL_AUTO_ANSWER_ON | Toggle Channel |
| 240 | DIAL_AUDIBLE_RING_ON | Toggle Channel |
| 238 | DIAL_OFF_HOOK_ON | fnUpdateCommandEx(vdvDev, CMD_DIALER_DIAL_NUMBER,sPhoneNumber) |
| 4 | CH_DIALER_ANSWER_CALL | fnUpdateCommandEx(vdvDev, CMD_DIALER_PICKUP_HANGUP,NULL_STR) |
| 208 | DIAL_FLASH_HOOK | PULSE[vdvDev,DIAL_FLASH_HOOK] |

### SpeedDial

STRUCTURE _SpeedDialStruct

{

INTEGER nIndex	// index of speed dial number, 1 to 50 CHAR sName[100]	// name of speed dial number

CHAR sNumber[100] // number to dial

}

### Update SpeedDial

fnSpeedDialStore(vdvDev, _SpeedDialStruct)

### Recall SpeedDial

fnSpeedDialRecall(vdv, _SpeedDialStruct)

# Appendix

## Object Types

| Object Types |  |  |  |
| --- | --- | --- | --- |
| Name | Mnemonic | Name | Mnemonic |
| AEC | AEC | Graphic EQ | GEQ |
| Ambient Noise Compensator Gap | ANG | High Pass Filter | HPF |
| Ambient Noise Compensator Non-Gap | ANN | Leveller | LEV |
| Analog Input Card | AIC | Limiter | LIM |
| Analog Output Card | AOC | Logic Source Select | LSS |
| Automixer Gain Sharing | AGS | Logic Source | LSO |
| Automixer Gated | AGT | Low Pass Filter | LPF |
| BLU Link Input Rx | BRX | Matrix Mixer | MXR |
| BLU Link Output Tx | BTX | Matrix Router | MRO |
| Compressor | CMP | Meter Peak | MPK |
| Crossover | CXO | Meter RMS | MRS |
| Dante Input Rx | DTI | Meter Trigger | MTG |
| Dante Output Tx | DTO | Mixer | MIX |
| Delay | DLY | Parametric EQ | PEQ |
| Digital Input Card | DIC | Phase Filter | PHF |
| Digital Output Card | DOC | Room Combine | RMC |
| Ducker | DUC | Source Matrix | SMX |
| Expander | EXP | Source Selector | SSL |
| Gain N-Input | GNI | Telephone Input Card | TIC |
| Gain | GAN | Parametric EQ | PEQ |
| Gain Timed | GTM | Tone Generator | TOG |
| Gate | GAT | VoIP Input Card | VIP |

## RoomCombine Room Object Mnemonics

| RoomCombine Room Object Mnemonics |  |  |
| --- | --- | --- |
| AA Label | Mnemonic | Values or Range |
| Group | GP | 0,1 |
| In Active Group | IA | ON (0), OFF |
| Master Gain | MG | -80dB to 10dB |
| Master Meter (Left/Mono) | ML | -30 - 20 dB (this is read only) |
| Master Mute | MM | unMuted (0)/Muted |
| Source Gain | SG | -80dB to 10dB |
| Source Mute | SM | unMuted (0)/Muted |
| BGM Gain | BG | -80dB to 10dB |
| BGM Mute | BM | unMuted (0)/Muted |
| BGM Source | BS | 0=none, 1-24 source |
| Master Meter (Right) | MR | -30 - 20 dB (this is read only) |

## Harman Pro Command Functions

| Harman Pro Command Functions |  |
| --- | --- |
| Function | Description |
| fnAddPots | Subscribe to Analog telephone device object |
| fnAddPotsEx | Subscribe to Analog telephone device object |
| fnAddVoip | Subscribe to VoIP telephone device object |
| fnAddVoipEx | Subscribe to VoIP telephone device object |
| fnBumpByPercentage | Send HProComm request to bump device object by +/- percentage |
| fnBumpByPercentageEx | Send HProComm request to bump device object by +/- percentage |
| fnPassthruCommand | Subscribe to object parameter with feedback using command |
| fnPassthruCommandEx | Subscribe to object parameter with feedback using command |
| fnRecallPrameterPreset | Recall a parameter preset; setup in Audio Architect |
| fnRegisterIPDevice | Connect to device via IP feedback on requested DPS |
| fnRegisterIPDeviceEx | Connect to device via IP feedback on requested DPS |
| fnRegisterRS232Device | Connect to device via IP feedback on requested DPS |
| fnRegisterRS232DeviceEx | Connect to device via IP feedback on requested DPS |
| fnRegisterRS232DeviceBaudRateEx | Connect to device via IP feedback on requested DPS and supply the optional baud rate |
| fnReportRoomCombine | Reports all items by room subscribed to |
| fnReportRoomCombinePartition | Subscribe to Room Combine partition, feedback using command |
| fnReportRoomCombineRoom | Reports all items by room subscribed to |
| fnReportSubscriptions | Reports all items by subscribed to |
| fnRoomCombineActiveGroupSubscribeChannel | Subscribe to Room Combine Active Group, feedback using channel |
| fnRoomCombineActiveGroupSubscribeChannelEx | Subscribe to Room Combine Active Group, feedback using channel |
| fnRoomCombineActiveGroupSubscribeCommand | Subscribe to Room Combine Active Group, feedback using command |
| fnRoomCombineActiveGroupSubscribeCommandEx | Subscribe to Room Combine Active Group, feedback using command |
| fnRoomCombineAddRoom | Add Room to Room Combine Object |
| fnRoomCombineAddRoomEx | Add Room to Room Combine Object |
| fnRoomCombinePartitionSubscribeChannel | Subscribe to Room Combine partition, feedback using channel |
| fnRoomCombinePartitionSubscribeChannelEx | Subscribe to Room Combine partition, feedback using channel |
| fnRoomCombinePartitionSubscribeCommand | Subscribe to Room Combine partition, feedback using command |
| fnRoomCombinePartitionSubscribeCommandEx | Subscribe to Room Combine partition, feedback using command |
| fnRoomCombineRegister | Register a Room Combine Object |
| fnRoomCombineRoomSubscribeChannel | Subscribe to room combine room object parameter with feedback on channel |
| fnRoomCombineRoomSubscribeChannelEx | Subscribe to room combine room object parameter with feedback on channel |
| fnRoomCombineRoomSubscribeCommand | Subscribe to room combine room object parameter with feedback using command |
| fnRoomCombineRoomSubscribeCommand | Subscribe to room combine room object parameter with feedback using command |
| fnRoomCombineRoomSubscribeCommandPercent | Subscribe to room combine room object parameter with feedback using command |
| fnRoomCombineRoomSubscribeCommandPercentEx | Subscribe to room combine room object parameter with feedback using command |
| fnRoomCombineRoomSubscribeLevel | Subscribe to room combine room for value feedback on level. Feedback values will be in the range of 0.00 - 255.00 |
| fnRoomCombineRoomSubscribeLevelEx | Subscribe to room combine room for value feedback on level. Feedback values will be in the range of 0.00 - 255.00 |
| fnRoomCombineRoomSubscribeLevelPrecent | Subscribe to room combine room for percentage feedback on level. Feedback values will be in the range of 0.00- 100.00 |
| fnRoomCombineRoomSubscribeLevelPrecentEx | Subscribe to room combine room for percentage feedback on level. Feedback values will be in the range of 0.00- 100.00 |

Harman Pro Command Functions (Cont.)

| Function | Description |
| --- | --- |
| fnRoomCombineRoomSubscribeLevelRange | Subscribe to room combine room for value feedback on level user specified range |
| fnRoomCombineRoomSubscribeLevelRangeEx | Subscribe to room combine room for value feedback on level user specified range |
| fnSpeedDialStore | Store name, number at given index in speed dial list |
| fnSpeedDialStoreEx | Store name, number at given index in speed dial list |
| fnSpeedDialStore | Store name, number at given index in speed dial list |
| fnSpeedDialStoreEx | Recall index in speed dial list |
| fnSubscribeChannel | Subscribe to object parameter with feedback on channel |
| fnSubscribeChannelEx | Subscribe channel |
| fnSubscribeCommand | Subscribe to object parameter with feedback using command |
| fnSubscribeCommandEx | Subscribe command |
| fnSubscribeCommandPercent | Subscribe for percentage feedback using command |
| fnSubscribeCommandPercentEx | Subscribe for percentage feedback using command |
| fnSubscribeLevel | Subscribe for value feedback on level. Feedback range of 0.00 - 255.00 |
| fnSubscribeLevelEx | Subscribe level |
| fnSubscribeLevelPercent | Subscribe for value feedback on level. Feedback range of 0.00 - 100.00 |
| fnSubscribeLevelPercentEx | Subscribe for value feedback on level. Feedback range of 0.00 - 100.00 |
| fnSubscribeLevelRange | Subscribe for value feedback on level user specified range |
| fnSubscribeLevelRangeEx | Subscribe level with range |
| fnUnsubscribe | Unsubscribe from object parameter |
| fnUpdateCommand | Update device object subscribed to using command |
| fnUpdateCommandEx | Update device object subscribed via command |

## Harman Pro Command Data Structures

| Harman Pro Command Data Structures |  |  |
| --- | --- | --- |
| Data Type | Name | Description |
| _AddTelephoneStruct |  |  |
| CHAR | sObjectID[8] | format n.n.n |
| INTEGER | nFBControlPort | virtual device's port to control item |
| INTEGER | nTelephoneLine | which line to control, 1 or 2 |
| INTEGER | nSpeedDialMax | number of speedDial slots to look for, default is 50, will conserve resources if less |
| _BumpByPercentStruct |  |  |
| CHAR | sObjectID[8] | format n.n.n |
| CHAR | sParameterID[6] | parameter id in 0xhex format |
| FLOAT | fBumpBy | range -100.00 to +100.00 |
| _PassthruCommandStruct |  |  |
| INTEGER | nFBControlPort | virtual device's port to control item |
| CHAR | sCommandPrefix[DUET_MAX_PARAM_LEN] | command prefix |
| CHAR | sPassthruData[DUET_MAX_PARAM_LEN] | passthru data to send |
| _RecallPreset |  |  |
| INTEGER | nPresetID | nPresetID to be recalled |
| _RegisterDeviceStruct |  |  |
| CHAR | sNodeID[6] | 0xhex format |
| DEV | vdvDevice | virtual device to control Object |
| CHAR | sIPAddrHostname[DUET_MAX_PARAM_LEN] | ipaddress/hostname |
| DEV | dvRS232Device | masters RS232 port |
| CHAR | sBaudRate[6] | baud rate - optional, default 115200 |

| Harman Pro Command Data Structures (Cont.) |  |  |
| --- | --- | --- |
| Data Type | Name | Description |
| _RoomCombineAddRoomStruct |  |  |
| CHAR | sRoomGroupAddr[6] | room group address in 0xhex format |
| CHAR | sRoomName[DUET_MAX_PARAM_LEN] | optional, room name |
| _RoomCombineActiveGroupSubscribeChannelStruct |  |  |
| CHAR | sActiveGroupAddr[6] | active group address in 0xhex format |
| INTEGER | nFBControlPort | virtual device's port to control item |
| INTEGER | nFBControlChannel | virtual device's channel to control item |
| _RoomCombineActiveGroupSubscribeCommandStruct |  |  |
| CHAR | sActiveGroupAddr[6] | active group address in 0xhex format |
| INTEGER | nFBControlPort | virtual device's port to control item |
| CHAR | sCommandPrefix[DUET_MAX_PARAM_LEN] | command prefix |
| _RoomCombinePartitionSubscribeChannelStruct |  |  |
| CHAR | sPartitionAddr[6] | partition address in 0xhex format |
| INTEGER | nFBControlPort | virtual device's port to control item |
| INTEGER | nFBControlChannel | virtual device's channel to control item |
| CHAR | sPartitionName[DUET_MAX_PARAM_LEN] | optional, partition name |
| _RoomCombinePartitionSubscribeCommandStruct |  |  |
| CHAR | sPartitionAddr[6] | partition address in 0xhex format |
| INTEGER | nFBControlPort | virtual device's port to control item |
| CHAR | sCommandPrefix[DUET_MAX_PARAM_LEN] | command prefix |
| CHAR | sPartitionName[DUET_MAX_PARAM_LEN] | optional, partition name |
| _RoomCombineRegisterStruct |  |  |
| CHAR | sObjectID[8] | format n.n.n |
| DEV | vdvRoomCombine | virtual device to control Room Combine Object |
| _RoomCombineSubscribeChannelStruct |  |  |
| CHAR | sRoomGroupAddr[6] | room group address in 0xhex format |
| CHAR | sRoomItemMnemonic[2] | see documentation for listing of RoomCombine Room Item Mnemonics |
| INTEGER | nFBControlPort | virtual device's port to control item |
| INTEGER | nFBControlChannel | virtual device's channel to control item |
| _RoomCombineSubscribeCommandStruct |  |  |
| CHAR | sRoomGroupAddr[6] | room group address in 0xhex format |
| CHAR | sRoomItemMnemonic[2] | see documentation for listing of RoomCombine Room Item Mnemonics |
| INTEGER | nFBControlPort | virtual device's port to control item |
| CHAR | sCommandPrefix[DUET_MAX_PARAM_LEN] | command prefix |
| _RoomCombineSubscribeLevelStruct |  |  |
| CHAR | sRoomGroupAddr[6] | room group address in 0xhex format |
| CHAR | sRoomItemMnemonic[2] | see documentation for listing of RoomCombine Room Item Mnemonics |
| INTEGER | nFBControlPort | virtual device's port to control item |
| INTEGER | nFBControlLevel | virtual device's level to control item |
| SINTEGER | nRangeMinimum | level range minimum |
| SINTEGER | nRangeMaximum | level range maximum |
| _SpeedDialStruct |  |  |
| INTEGER | nIndex | index of speed dial number, 1 to 50 |
| CHAR | sName[100] | name of speed dial number |
| CHAR | sNumber[100] | number to dial |

| Harman Pro Command Data Structures (Cont.) |  |  |
| --- | --- | --- |
| Data Type | Name | Description |
| _SubscribeChannelStruct |  |  |
| CHAR | sObjectID[8] | Format n.n.n |
| CHAR | sObjectType[4] | see documentation for listing of Object Types |
| CHAR | sParameterID[6] | parameter id in 0xhex format |
| INTEGER | nFBControlPort | virtual device's port to control item |
| INTEGER | nFBControlChannel | virtual device's channel to control item |
| _SubscribeCommandStruct |  |  |
| CHAR | sObjectID[8] | format n.n.n |
| CHAR | sObjectType[4] | see documentation for listing of Object Types |
| CHAR | sParameterID[6] | parameter id in 0xhex format |
| INTEGER | nFBControlPort | virtual device's port to control item |
| CHAR | sCommandPrefix[DUET_MAX_PARAM_LEN] | command prefix |
| SINTEGER | nRangeMinimum | level range minimum |
| SINTEGER | nRangeMaximum | level range maximum |
| _SubscribeLevelStruct |  |  |
| CHAR | sObjectID[8] | format n.n.n |
| CHAR | sObjectType[4] | see documentation for listing of Object Types |
| CHAR | sParameterID[6] | parameter id in 0xhex format |
| INTEGER | nFBControlPort | virtual device's port to control item |
| INTEGER | nFBControlLevel | virtual device's level to control item |
| SINTEGER | nRangeMinimum | level range minimum |
| SINTEGER | nRangeMaximum | level range maximum |
| _UnSubscribeStruct |  |  |
| CHAR | sObjectID[8] | format n.n.n |
| CHAR | sParameterID[6] | parameter id in 0xhex format |
| _UpdateCommandValueStruct |  |  |
| CHAR | sCommandPrefix[DUET_MAX_PARAM_LEN] | command prefix |
| CHAR | sValue[DUET_MAX_PARAM_LEN] | update to value |

## Dialer NetLinx Module Implemented Buttons

### VoIP TP

| VoIP TP |  |  |
| --- | --- | --- |
| Button Name | Channel | Address |
| AccountError |  | 3 |
| AnswerCall | 4 | 4 |
| Audible Ring | 240 |  |
| Auto Answer 1 Ring status | 3177 |  |
| Auto Answer 2 Rings status | 3178 |  |
| Auto Answer 3 Rings status | 3179 |  |
| Auto Answer Off | 3176 |  |
| AutoAnswer | 239 |  |
| Backspace | 3172 |  |
| Call Error |  | 2 |
| CallerID | 1 | 1 |
| Delete | 3171 |  |
| Dial/HangUp | 202 |  |
| Dialer # | 93 |  |
| Dialer #0 | 10 |  |
| Dialer #1 | 11 |  |
| Dialer #2 | 12 |  |
| Dialer #3 | 13 |  |
| Dialer #4 | 14 |  |
| Dialer #5 | 15 |  |
| Dialer #6 | 16 |  |
| Dialer #7 | 17 |  |
| Dialer #8 | 18 |  |
| Dialer #9 | 19 |  |
| Dialer Pause | 94 |  |
| Dialer Splat | 91 |  |
| Dial Number |  | 3170 |
| Hold | 85 |  |
| Incoming Call | 3174 |  |
| Line Status | 3173 |  |
| PickUp/HangUp | 238 | 4 |
| Privacy | 146 |  |
| Redial | 201 |  |
| Reject | 61 |  |
| SPD Clear Name |  | 1:6 |
| SPD Clear Number | 22 |  |
| SPD Edit Name | 11 |  |
| SPD Edit Number | 21 |  |
| SPD Name Text |  | 10 |
| SPD Number Text |  | 20 |
| SpeedDial Name1 |  | 601 |
| SpeedDial Name2 |  | 602 |
| SpeedDial Name3 |  | 603 |
| SpeedDial Name4 |  | 604 |
| SpeedDial Name5 |  | 605 |
| SpeedDial Name6 |  | 606 |

| VoIP TP (Cont.) |  |  |
| --- | --- | --- |
| Button Name | Channel | Address |
| SpeedDial Name7 |  | 607 |
| SpeedDial Name8 |  | 608 |
| SpeedDial Name9 |  | 609 |
| SpeedDial Name10 |  | 610 |
| SpeedDial Name11 |  | 611 |
| SpeedDial Name12 |  | 612 |
| SpeedDial Name13 |  | 613 |
| SpeedDial Name14 |  | 614 |
| SpeedDial Name15 |  | 615 |
| SpeedDial Name16 |  | 616 |
| SpeedDial Number1 |  | 701 |
| SpeedDial Number2 |  | 702 |
| SpeedDial Number3 |  | 703 |
| SpeedDial Number4 |  | 704 |
| SpeedDial Number5 |  | 705 |
| SpeedDial Number6 |  | 706 |
| SpeedDial Number7 |  | 707 |
| SpeedDial Number8 |  | 708 |
| SpeedDial Number9 |  | 709 |
| SpeedDial Number10 |  | 710 |
| SpeedDial Number11 |  | 711 |
| SpeedDial Number12 |  | 712 |
| SpeedDial Number13 |  | 713 |
| SpeedDial Number14 |  | 714 |
| SpeedDial Number15 |  | 715 |
| SpeedDial Number16 |  | 716 |
| SpeedDial Recall1 | 801 |  |
| SpeedDial Recall2 | 802 |  |
| SpeedDial Recall3 | 803 |  |
| SpeedDial Recall4 | 804 |  |
| SpeedDial Recall5 | 805 |  |
| SpeedDial Recall6 | 806 |  |
| SpeedDial Recall7 | 807 |  |
| SpeedDial Recall8 | 808 |  |
| SpeedDial Recall9 | 809 |  |
| SpeedDial Recall10 | 810 |  |
| SpeedDial Recall11 | 811 |  |
| SpeedDial Recall12 | 812 |  |
| SpeedDial Recall13 | 813 |  |
| SpeedDial Recall14 | 814 |  |
| SpeedDial Recall15 | 815 |  |
| SpeedDial Recall16 | 816 |  |
| SpeedDial Store1 | 501 |  |
| SpeedDial Store2 | 502 |  |
| SpeedDial Store3 | 503 |  |
| SpeedDial Store4 | 504 |  |
| SpeedDial Store5 | 505 |  |
| SpeedDial Store6 | 506 |  |
| SpeedDial Store7 | 507 |  |

| VoIP TP (Cont.) |  |  |
| --- | --- | --- |
| Button Name | Channel | Address |
| SpeedDial Store8 | 508 |  |
| SpeedDial Store9 | 509 |  |
| SpeedDial Store10 | 510 |  |
| SpeedDial Store11 | 511 |  |
| SpeedDial Store12 | 512 |  |
| SpeedDial Store13 | 513 |  |
| SpeedDial Store14 | 514 |  |
| SpeedDial Store15 | 515 |  |
| SpeedDial Store16 | 516 |  |
| Volume Down | 25 |  |
| Volume Up | 24 |  |

### POTS TP

| POTS TP |  |  |
| --- | --- | --- |
| Button Name | Channel | Address |
| AnswerCall | 4 | 4 |
| Auto Answer 1 Ring status | 3177 |  |
| Auto Answer 2 Rings status | 3178 |  |
| Auto Answer 3 Rings status | 3179 |  |
| Auto Answer 4 Rings status | 3180 |  |
| Auto Answer 5 Rings status | 3181 |  |
| Auto Answer 6 Rings status | 3182 |  |
| Auto Answer 7 Rings status | 3183 |  |
| Auto Answer 8 Rings status | 3184 |  |
| Auto Answer 9 Rings status | 3185 |  |
| Auto Answer 10 Rings status | 3186 |  |
| Auto Answer Off | 3176 |  |
| AutoAnswer | 239 |  |
| Backspace | 3172 |  |
| Delete | 3171 |  |
| Dial/HangUp | 202 |  |
| Dialer # | 93 |  |
| Dialer #0 | 10 |  |
| Dialer #1 | 11 |  |
| Dialer #2 | 12 |  |
| Dialer #3 | 13 |  |
| Dialer #4 | 14 |  |
| Dialer #5 | 15 |  |
| Dialer #6 | 16 |  |
| Dialer #7 | 17 |  |
| Dialer #8 | 18 |  |
| Dialer #9 | 19 |  |
| Dialer + | 3163 |  |
| Dialer Pause | 94 |  |
| Dialer Splat | 91 |  |
| Dial Number |  | 3170 |
| Flash Hook | 208 |  |
| Incoming Call | 3174 |  |
| Line Status | 3173 |  |

| POTS TP (Cont.) |  |  |
| --- | --- | --- |
| Button Name | Channel | Address |
| PickUp/HangUp | 238 | 4 |
| Privacy | 146 |  |
| Redial | 201 |  |
| SpeedDial Clear Number | 22 |  |
| SpeedDial Edit Number | 21 |  |
| SpeedDial Number Text |  | 20 |
| SpeedDial Number1 |  | 701 |
| SpeedDial Number2 |  | 702 |
| SpeedDial Number3 |  | 703 |
| SpeedDial Number4 |  | 704 |
| SpeedDial Number5 |  | 705 |
| SpeedDial Number6 |  | 706 |
| SpeedDial Number7 |  | 707 |
| SpeedDial Number8 |  | 708 |
| SpeedDial Number9 |  | 709 |
| SpeedDial Number10 |  | 710 |
| SpeedDial Number11 |  | 711 |
| SpeedDial Number12 |  | 712 |
| SpeedDial Number13 |  | 713 |
| SpeedDial Number14 |  | 714 |
| SpeedDial Number15 |  | 715 |
| SpeedDial Number16 |  | 716 |
| SpeedDial Recall1 | 801 |  |
| SpeedDial Recall2 | 802 |  |
| SpeedDial Recall3 | 803 |  |
| SpeedDial Recall4 | 804 |  |
| SpeedDial Recall5 | 805 |  |
| SpeedDial Recall6 | 806 |  |
| SpeedDial Recall7 | 807 |  |
| SpeedDial Recall8 | 808 |  |
| SpeedDial Recall9 | 809 |  |
| SpeedDial Recall10 | 810 |  |
| SpeedDial Recall11 | 811 |  |
| SpeedDial Recall12 | 812 |  |
| SpeedDial Recall13 | 813 |  |
| SpeedDial Recall14 | 814 |  |
| SpeedDial Recall15 | 815 |  |
| SpeedDial Recall16 | 816 |  |
| SpeedDial Store1 | 501 |  |
| SpeedDial Store2 | 502 |  |
| SpeedDial Store3 | 503 |  |
| SpeedDial Store4 | 504 |  |
| SpeedDial Store5 | 505 |  |
| SpeedDial Store6 | 506 |  |
| SpeedDial Store7 | 507 |  |
| SpeedDial Store8 | 508 |  |
| SpeedDial Store9 | 509 |  |
| SpeedDial Store10 | 510 |  |
| SpeedDial Store11 | 511 |  |

| POTS TP (Cont.) |  |  |
| --- | --- | --- |
| Button Name | Channel | Address |
| SpeedDial Store12 | 512 |  |
| SpeedDial Store13 | 513 |  |
| SpeedDial Store14 | 514 |  |
| SpeedDial Store15 | 515 |  |
| SpeedDial Store16 | 516 |  |
| Volume Down | 25 |  |
| Volume Up | 24 |  |

© 2020 Harman. All rights reserved. Metreau, NetLinx, AMX, AV FOR AN IT WORLD, HARMAN, and their respective logos are registered trademarks of HARMAN. Oracle, Java and any other company or brand name referenced may be trademarks/registered trademarks of their respective companies. AMX does not assume responsibility for errors or omissions. AMX also reserves the right to alter specifications without prior notice at any time.

The AMX Warranty and Return Policy and related documents can be viewed/downloaded at www.amx.com.

3000 RESEARCH DRIVE, RICHARDSON, TX 75082 AMX.com | 800.222.0193 | 469.624.8000 | +1.469.624.7400 | fax 469.624.7153

AMX (UK) LTD, AMX by HARMAN - Unit C, Auster Road, Clifton Moor, York, YO30 4GD United Kingdom • +44 1904-343-100 • www.amx.com/eu/

##### Last Revised: 7/31/2020